MERCHANT SHIPPING (PREVENTION OF POLLUTION BY SEWAGE) REGULATION

- CHAPTER 413K

LONG TITLE

Empowering section VerDate:02/02/2007

(Cap 413, section 3(2))

[2 February 2007]  L.N. 287 of 2006

(L.N. 7 of 2005)

SECT 1

(Omitted as spent) VerDate:02/02/2007

PART 1

GENERAL PROVISIONS

SECT 2

Interpretation VerDate:02/02/2007

In this Regulation, unless the context otherwise requires-

"additional survey" (é™„åŠ æª¢é©—) means an additional survey referred to in

section 26(1);

"Annex IV" (é™„ä»¶IV) means the revised Annex IV to the Convention adopted by

the International Maritime Organization by resolution MEPC.115(51), as

from time to time revised or amended by the revision or amendment that

applies to Hong Kong;

"company" (å…¬å¸), in relation to a ship, means-

(a)	the owner of the ship; or

(b)	any other person who has assumed responsibility for the

operation of the ship, and on assuming such responsibility has agreed to

take over all the duties and responsibilities imposed in respect of the

ship by the Convention;

"Convention" (ã€Šå…¬ç´„ã€‹) means the International Convention for the Prevention

of Pollution from Ships, 1973, as modified by the Protocol of 1978

relating to that Convention;

"Convention country" (å…¬ç´„åœ‹) means a country which is a party to the

Convention;

"from the nearest land" (è·é›¢æœ€è¿‘é™¸åœ°)-

(a)	in relation to all land other than the part of Australia

mentioned in paragraph (b), means from the low-water line along the coast

as marked on large-scale charts officially recognized by the coastal state;

(b)	in relation to the part of the North-eastern coast of Australia

which lies between the points 11o 00'S, 142o 08'E and 24o 42'S, 153o 15'E,

means from the straight lines joining consecutively the following points-

(i)	11o 00'S, 142o 08'E;

(ii)	10o 35'S, 141o 55'E;

(iii)	10o 00'S, 142o 00'E;

(iv)	9o 10'S, 143o 52'E;

(v)	9o 00'S, 144o 30'E;

(vi)	10o 41'S, 145o 00'E;

(vii)	13o 00'S, 145o 00'E;

(viii)	15o 00'S, 146o 00'E;

(ix)	17o 30'S, 147o 00'E;

(x)	21o 00'S, 152o 55'E;

(xi)	24o 30'S, 154o 00'E; and

(xii)	24o 42'S, 153o 15'E;

"Government surveyor" (æ”¿åºœé©—èˆ¹å¸«) means the person appointed under section 4

to be a Government surveyor;

"holding tank" (é›†å˜è‰™) means a tank used for the collection and storage of

sewage;

"Hong Kong ship" (é¦™æ¸¯èˆ¹èˆ¶) means a ship registered under the Merchant

Shipping (Registration) Ordinance (Cap 415);

"initial survey" (åˆæ¬¡æª¢é©—) means an initial survey referred to in section

9(2)(a)(i);

"international voyage" (åœ‹éš›èˆªç¨‹) means a voyage from a Convention country to

a port outside that Convention country, or vice versa;

"ISPP Certificate" (åœ‹éš›é˜²æ±¡æ°´è‰æ›¸) means-

(a)	in relation to a Hong Kong ship-

(i)	a sewage certificate;

(ii)	an International Sewage Pollution Prevention Certificate

issued by a recognized organization in conformity with Annex IV to the

Convention; or

(iii)	an International Sewage Pollution Prevention Certificate

issued by any Convention country on behalf of the Director in respect of a

Hong Kong ship in conformity with Annex IV to the Convention; or

(b)	in relation to a non-Hong Kong ship, an International Sewage

Pollution Prevention Certificate issued in conformity with Annex IV to the

Convention;

"non-Hong Kong ship" (éžé¦™æ¸¯èˆ¹èˆ¶) means a ship other than a Hong Kong ship;

"recognized organization" (èªå¯æ©Ÿæ§‹) means an organization recognized under

section 5;

"renewal survey" (çºŒè‰æª¢é©—) means a renewal survey referred to in section

9(2)(a)(ii);

"sewage" (æ±¡æ°´) means-

(a)	drainage or other wastes from any form of toilet or urinal;

(b)	drainage from medical premises via wash basins, wash tubs or

scuppers located in such premises;

(c)	drainage from spaces containing living animals; or

(d)	other waste waters when mixed with any drainage or wastes

specified in paragraph (a), (b) or (c) or any combination of them;

"sewage certificate" (æ±¡æ°´è‰æ›¸) means an International Sewage Pollution

Prevention Certificate issued by the Director under section 9;

"surveyor" (é©—èˆ¹å¸«) means-

(a)	a Government surveyor; or

(b)	a recognized organization.

SECT 3

Application and exemption VerDate:02/02/2007

(1)Subject to subsections (2), (3) and (4), this Regulation

applies to a ship that-

(a)	is engaged in international voyages;

(b)	is of 400 gross tonnage or above, or is of less than 400 gross

tonnage but is certified to carry more than 15 persons; and

(c)	is-

(i)	a Hong Kong ship wherever it may be; or

(ii)	a non-Hong Kong ship within the waters of Hong Kong.

(2)This Regulation does not apply to a ship-

(a)	the building contract for which is placed before 27 September

2003, or in the absence of a building contract, the keel of which is laid,

or which is at a similar stage of construction, before 27 September 2003;

and

(b)	the delivery of which is before 27 September 2006.

(3)This Regulation does not apply to-

(a)	warships;

(b)	naval auxiliaries; and

(c)	other ships owned or operated by a government and used only on

government non-commercial service.

(4)The Director may exempt any ship or class or description of

ships from any of the requirements of this Regulation on such conditions

as he may specify and he may, subject to giving reasons, alter or cancel

any exemption so granted.

(5) Subsection (2) expires at midnight on 26 September 2008.

SECT 4

Appointment of Government surveyors VerDate:02/02/2007

The Director may appoint a person to be a Government surveyor for the

purposes of this Regulation.

SECT 5

Director may recognize organizations to survey ships and issue and endorse certificates VerDate:02/02/2007

(1) The Director may recognize any organization for the purposes of-

(a)	carrying out surveys in respect of Hong Kong ships in

conformity with Annex IV to the Convention;

(b)	issuing International Sewage Pollution Prevention Certificates

in respect of Hong Kong ships in conformity with that Annex; and

(c)	making endorsements on ISPP Certificates issued in respect of

Hong Kong ships in conformity with that Annex.

(2) For the purposes of this Regulation, an endorsement made on an

ISPP Certificate by a recognized organization in conformity with Annex IV

to the Convention has the same effect as an endorsement made by the

Director under section 14.

SECT 6

Director may request Convention countries to survey ships and issue or endorse certificates VerDate:02/02/2007

(1) The Director may request any Convention country-

(a)	to carry out a survey in respect of a Hong Kong ship on his

behalf in conformity with Annex IV to the Convention; and

(b)	to issue an International Sewage Pollution Prevention

Certificate in respect of the ship, or to make an endorsement on the ISPP

Certificate issued in respect of the ship, on his behalf in conformity

with that Annex.

(2) For the purposes of this Regulation, an endorsement made on an

ISPP Certificate by a Convention country on behalf of the Director in

conformity with Annex IV to the Convention has the same effect as an

endorsement made by the Director under section 14.

SECT 7

Equivalents VerDate:02/02/2007

Where this Regulation requires that a particular fitting or equipment

shall be fitted in a ship, the Director may allow any other fitting or

equipment to be fitted in the ship if he is satisfied that such fitting or

equipment is at least as effective as that required by this Regulation.

SECT 8

Prohibition on engaging Hong Kong ships in international voyages without ISPP Certificates VerDate:02/02/2007

PART 2

PROHIBITION ON ENGAGING HONG KONG SHIPS IN

INTERNATIONAL VOYAGES WITHOUT ISPP

CERTIFICATES

A Hong Kong ship to which this Regulation applies shall not be engaged in

international voyages unless there is in force in respect of the ship an

ISPP Certificate.

SECT 9

Applications for issue of International Sewage Pollution Prevention Certificates VerDate:02/02/2007

PART 3

ISSUE OF INTERNATIONAL SEWAGE POLLUTION PREVENTION

CERTIFICATES AND RELATED MATTERS

(1) A company of a Hong Kong ship may apply to the Director for an

International Sewage Pollution Prevention Certificate in respect of the

ship. An application is to be accompanied by the prescribed fee in

relation to the issue of the International Sewage Pollution Prevention

Certificate.

(2) The Director shall not issue an International Sewage Pollution

Prevention Certificate in respect of a Hong Kong ship unless the Director

is satisfied-

(a)	where the ship-

(i)	is one in respect of which an ISPP Certificate has never been

issued, that an initial survey of the ship has been carried out in

accordance with section 10; or

(ii)	is one in respect of which an ISPP Certificate has been

issued, that a renewal survey of the ship has been carried out in

accordance with section 10; and

(b)	that, on the evidence of a declaration of survey forwarded to

the Director under section 10(4), the structure, equipment, systems,

fittings, arrangements and material of the ship comply with the

requirements under the Schedule and the equipment and associated piping

system are in good working order.

(3) An International Sewage Pollution Prevention Certificate issued

under this section is valid for a period specified by the Director in

accordance with section 12 or 17.

SECT 10

Initial surveys and renewal surveys VerDate:02/02/2007

(1) An initial survey or renewal survey of a ship shall be carried

out by a surveyor.

(2) An initial survey of a ship shall include a complete survey of

the structure, equipment, systems, fittings, arrangements and material of

the ship.

(3) A renewal survey of a ship shall, except where section 12(2) or

(4)(b), 15, 16 or 17(2) is applicable, be carried out on a date not

exceeding 5 years from the date of completion of the preceding initial

survey or renewal survey, as may be applicable.

(4) If, after having carried out an initial survey or renewal

survey in respect of a ship, the surveyor is satisfied that the structure,

equipment, systems, fittings, arrangements and material of the ship comply

with the requirements under the Schedule and the equipment and associated

piping system are in good working order, he shall-

(a)	make a declaration of survey to the effect that he is so

satisfied; and

(b)	forward the declaration of survey to the Director.

SECT 11

Form of sewage certificates VerDate:02/02/2007

(1) The Director may specify the form of a sewage certificate.

(2) The form specified under subsection (1) shall correspond to the

Form of Certificate set out in the Appendix to Annex IV to the Convention.

SECT 12

Duration of sewage certificates VerDate:02/02/2007

(1) Subject to subsections (2) and (4), a sewage certificate issued

as a result of an initial survey or renewal survey is valid for a period

of not exceeding 5 years from the date of completion of the survey.

(2) If the renewal survey carried out in respect of a Hong Kong

ship is completed within 3 months before the expiry date of the existing

ISPP Certificate in force in respect of the ship, the new sewage

certificate issued as a result of the survey is valid from the date of

completion of the survey to a date not exceeding 5 years from the expiry

date of the existing ISPP Certificate.

(3) If the renewal survey carried out in respect of a Hong Kong

ship is completed more than 3 months before the expiry date of the

existing ISPP Certificate in force in respect of the ship, the new sewage

certificate issued as a result of the survey is valid from the date of

completion of the survey to a date not exceeding 5 years from the date of

completion of the survey.

(4) If the renewal survey carried out in respect of a Hong Kong

ship is completed after the expiry date of the existing ISPP Certificate

in force in respect of the ship, the new sewage certificate issued as a

result of the survey-

(a)	subject to paragraph (b), is valid from the date of completion

of the survey to a date not exceeding 5 years from the expiry date of the

existing ISPP Certificate; or

(b)	where the Director considers appropriate, is valid from the

date of completion of the survey to a date not exceeding 5 years from the

date of completion of the survey.

SECT 13

Extension of period of validity VerDate:02/02/2007

(1) If an ISPP Certificate issued in respect of a Hong Kong ship is

valid for a period of less than 5 years, the Director may on the

application of the company or the master of the ship extend the period of

validity of the Certificate beyond the expiry date.

(2) Subsection (1) does not empower the Director to extend the

period of validity of an ISPP Certificate in such a way that the

Certificate will be valid for more than 5 years.

SECT 14

Extension of period of validity where new sewage certificates cannot be issued, etc. before expiry of existing ISPP Certificates VerDate:02/02/2007

If, on an application for a sewage certificate, the Director is satisfied

as to the matters set out in section 9(2)(a)(ii) and (b) but the new

sewage certificate cannot be issued or placed on board the ship concerned

before the expiry date of the existing ISPP Certificate in force in

respect of the ship, the Director may-

(a)	extend the period of validity of the existing ISPP Certificate

for a further period of not exceeding 5 months from the expiry date of the

existing ISPP Certificate; and

(b)	endorse on the existing ISPP Certificate a statement to that

effect.

SECT 15

Extension of period of validity where ships are not in ports in which ships are to be surveyed VerDate:02/02/2007

(1) If an ISPP Certificate issued in respect of a Hong Kong ship

expires when the ship is not in a port in which the ship is to be

surveyed, the company or the master of the ship may apply to the Director

for an extension of the period of validity of the Certificate.

(2) The Director shall not grant an extension of the period of

validity of an ISPP Certificate unless-

(a)	it is for the purpose of allowing the ship to complete its

voyage to a port in which the ship is to be surveyed; and

(b)	the Director considers that it is proper and reasonable to do

so.

(3) Subject to subsection (4), the Director may, on granting an

extension of the period of validity of an ISPP Certificate, extend the

period of validity of the Certificate for a period of not exceeding 3

months from the expiry date of the Certificate.

(4) If the period of validity of an ISPP Certificate has been

extended under this section, the Certificate shall, notwithstanding such

extension, expire when the ship completes its voyage to a port in which

the ship is to be surveyed.

SECT 16

Extension of period of validity where ships are on short voyages VerDate:02/02/2007

Where a Hong Kong ship in respect of which an ISPP Certificate has been

issued is engaged in short voyages and the period of validity of the

Certificate has not been extended under section 13(1) or 15(3), the

Director may on the application of the company or the master of the ship

extend the period of validity of the Certificate for a period of not

exceeding 1 month from the expiry date of the Certificate.

SECT 17

Duration of new sewage certificates where period of validity is extended under section 15 or 16 VerDate:02/02/2007

(1) If the period of validity of an existing ISPP Certificate

issued in respect of a ship has been extended under section 15 or 16, the

new sewage certificate issued in respect of the ship as a result of a

renewal survey is valid from the date of completion of the survey to a

date not exceeding 5 years from the expiry date of the existing ISPP

Certificate before the extension was granted.

(2) Notwithstanding subsection (1), where the Director considers

appropriate, a new sewage certificate may be valid from the date of

completion of the relevant survey to a date not exceeding 5 years from the

date of completion of the survey.

SECT 18

Where ISPP Certificates cease to be valid VerDate:02/02/2007

An ISPP Certificate issued in respect of a Hong Kong ship ceases to be

valid-

(a)	if an additional survey of the ship is not carried out within

the period specified by the Director under section 26(1);

(b)	if the ship is transferred to the registry of a place outside

Hong Kong; or

(c)	if any alteration has been made, without the approval of the

Director, to the structure, equipment, systems, fittings, arrangements or

material of the ship covered by the initial or renewal survey leading to

the issue, or additional survey leading to the endorsement, of the

Certificate, other than an alteration that constitutes a direct

replacement of such equipment or fittings.

SECT 19

Certified true copies of sewage certificates VerDate:02/02/2007

The company of a Hong Kong ship in respect of which a sewage certificate

has been issued may apply to the Director for the issue of a certified

true copy of the certificate. An application is to be accompanied by the

prescribed fee in relation to the issue of the certified true copy.

SECT 20

Alteration made to sewage certificates VerDate:02/02/2007

(1) The company of a Hong Kong ship in respect of which a sewage

certificate has been issued may request the Director to alter any

particulars contained in the certificate.

(2) The Director may refuse to make the alteration requested if he

considers the alteration to be made is material.

(3) If the Director agrees to make the alteration, he shall, upon

receipt of the prescribed fee in relation to the alteration, alter the

sewage certificate accordingly.

SECT 21

Interpretation of Part 4 VerDate:02/02/2007

PART 4

DUTIES OF COMPANIES AND MASTERS, ADDITIONAL

SURVEYS AND SUSPENSION OF VALIDITY OF ISPP

CERTIFICATES, ETC.

For the purposes of this Part, a serious accident occurs to a ship or

there is a material defect in a ship if the accident or defect

substantially affects-

(a)	the safety of the ship and persons on board the ship;

(b)	the ship to the extent that the ship is not fit to proceed to

sea without presenting an unreasonable threat of harm to the marine

environment; or

(c)	the efficiency or completeness of the equipment of the ship-

(i)	where the ship is a Hong Kong ship, set out in the Schedule;

(ii)	where the ship is a non-Hong Kong ship, set out in the

provisions of Annex IV to the Convention that are equivalent to the

provisions of the Schedule.

SECT 22

Duty to keep ISPP Certificates on ships VerDate:02/02/2007

An ISPP Certificate issued in respect of a ship shall be kept on board the

ship and be available for inspection by a Government surveyor at all

reasonable time.

SECT 23

Duty to maintain condition of ships VerDate:02/02/2007

The condition of a Hong Kong ship in respect of which an ISPP Certificate

has been issued, and its equipment, shall be maintained so as to comply

with the requirements under the Schedule to ensure that the ship in all

respects remains fit to proceed to sea without presenting an unreasonable

threat of harm to the marine environment.

SECT 24

Duty to report defects of ships, etc. VerDate:02/02/2007

(1) Where a serious accident occurs to, or a material defect is

discovered in, a ship in respect of which an ISPP Certificate has been

issued, the company or the master of the ship-

(a)	shall immediately report the accident or defect to the

Director; and

(b)	where the ship is a Hong Kong ship that is in a port of any

Convention country, shall immediately report the accident or defect to the

appropriate authority of that country.

(2) On receiving a report under subsection (1)(a), the Director

may, where the ship concerned is a Hong Kong ship, cause an investigation

to be initiated and determine whether a survey is necessary.

SECT 25

Actions against ships that are not fit to proceed to sea without presenting unreasonable threat of harm to marine environment, etc. VerDate:02/02/2007

(1) If the Director has reasonable ground to believe that-

(a)	the condition of a ship in respect of which an ISPP Certificate

has been issued, or its equipment, does not correspond substantially with

the particulars contained in the Certificate; or

(b)	the condition of a ship in respect of which an ISPP Certificate

has been issued is such that the ship is not fit to proceed to sea without

presenting an unreasonable threat of harm to the marine environment,

he may appoint a surveyor to carry out an investigation and report to him.

(2) Where the Director appoints a surveyor under subsection (1),

the Director shall inform the company or the master of the ship concerned

of the appointment.

(3) If, on receiving a report under subsection (1) concerning a

Hong Kong ship, the Director is satisfied that-

(a)	the matter referred to in subsection (1)(a) or (b) is

established; and

(b)	the corrective action which in the opinion of the surveyor is

required has not been taken within the period specified by the surveyor

or, in the absence of such a specified period, a reasonable period,

the Director may, by notice in writing to the company of the ship, suspend

the validity of the ISPP Certificate issued in respect of the ship.

(4) On receiving a notice under subsection (3), the company of the

ship concerned shall forthwith cause to be delivered to the Director the

ISPP Certificate issued in respect of the ship.

(5) Where the validity of an ISPP Certificate issued in respect of

a Hong Kong ship has been suspended under subsection (3), the company of

the ship may, after the corrective action referred to in subsection (3)(b)

has been taken in respect of the ship, apply to the Director for the

restoration of the validity of the Certificate.

(6) On receiving an application for the restoration of the validity

of an ISPP Certificate under subsection (5), the Director shall, if he is

satisfied that the corrective action referred to in subsection (3)(b) has

been taken in respect of the Hong Kong ship concerned after the

suspension, restore the validity of the Certificate.

(7) Where the Director restores the validity of an ISPP Certificate

under subsection (6), he shall notify the company of the Hong Kong ship

concerned in writing of the restoration and return the Certificate to the

company.

SECT 26

Additional surveys VerDate:02/02/2007

(1) If-

(a)	the Director has reasonable ground to believe that section 23

is not complied with in respect of a Hong Kong ship;

(b)	after an ISPP Certificate has been issued, or endorsed under

subsection (4), in respect of a Hong Kong ship, any alteration has been

made, with the approval of the Director, to the structure, equipment,

systems, fittings, arrangements or material of the ship covered by the

survey leading to the issue or endorsement of the Certificate;

(c)	the Director determines under section 24(2) that it is

necessary to carry out a survey in respect of a Hong Kong ship;

(d)	the Director has reasonable ground to believe that, after an

ISPP Certificate has been issued, or endorsed under subsection (4), in

respect of a Hong Kong ship, important repairs or renewals have been made

to the ship; or

(e)	the Director has reasonable ground to believe that a serious

accident has occurred to a Hong Kong ship in respect of which an ISPP

Certificate has been issued, or that there is a material defect in the

ship,

the Director may require that an additional survey, either general or

partial as the Director thinks fit, be carried out in respect of the ship

by a surveyor within a reasonable period specified by the Director.

(2) Where the Director requires that an additional survey be

carried out in respect of a Hong Kong ship under subsection (1), he shall

inform the company or the master of the ship of the requirement.

(3) If, after having carried out an additional survey in respect of

a Hong Kong ship, the surveyor is satisfied that-

(a)	the ship in all respects complies with the requirements under

the Schedule; and

(b)	the ship is fit to proceed to sea without presenting an

unreasonable threat of harm to the marine environment,

he shall make a declaration of survey to the effect that he is so

satisfied and forward the declaration of survey to the Director.

(4) If, on receiving a declaration of survey in respect of a Hong

Kong ship under subsection (3), the Director is satisfied-

(a)	that an additional survey in respect of the ship has been

carried out; and

(b)	that, on the evidence of the declaration of survey, the ship in

all respects complies with the requirements under the Schedule and is fit

to proceed to sea without presenting an unreasonable threat of harm to the

marine environment,

he shall cause to be endorsed on the ISPP Certificate issued in respect of

the ship a statement to the effect that he is so satisfied.

SECT 27

Cancellation of ISPP Certificates VerDate:02/02/2007

(1) The Director may, by notice in writing to the company of a Hong

Kong ship, cancel an ISPP Certificate issued or endorsed in respect of the

ship if he has reasonable ground to believe that the Certificate was

issued or endorsed on the basis of false or erroneous information.

(2) The Director shall give reasons for cancelling an ISPP

Certificate.

(3) On receiving a notice under subsection (1), the company of the

ship concerned shall forthwith cause to be delivered to the Director the

ISPP Certificate issued in respect of the ship.

SECT 28

Restriction on discharge of sewage into sea VerDate:02/02/2007

PART 5

RESTRICTION ON DISCHARGE OF SEWAGE INTO SEA

(1) No sewage may be discharged into the sea from a ship to which

this Regulation applies unless-

(a)	in the case where the sewage is comminuted and disinfected, all

of the following conditions are complied with-

(i)	the ship is discharging the sewage using a system referred to

in section 1(b) of the Schedule or a sewage comminuting and disinfecting

system specified in the ISPP Certificate issued in respect of the ship, at

a distance of more than 3 nautical miles from the nearest land;

(ii)	the sewage that has been stored in holding tanks is not

discharged instantaneously;

(iii)	the effluent does not produce visible floating solids nor

cause discoloration of the surrounding water; and

(iv)	the ship is proceeding en route at not less than 4 knots;

(b)	in the case where the sewage is not comminuted or disinfected,

all of the following conditions are complied with-

(i)	the sewage is discharged at a distance of more than 12 nautical

miles from the nearest land;

(ii)	the sewage that has been stored in holding tanks is not

discharged instantaneously;

(iii)	the effluent does not produce visible floating solids nor

cause discoloration of the surrounding water; and

(iv)	the ship is proceeding en route at not less than 4 knots; or

(c)	in the case where the ship has in operation a sewage treatment

plant referred to in section 1(a) of the Schedule or a sewage treatment

plant specified in the ISPP Certificate issued in respect of the ship-

(i)	the test results of the plant are laid down in the ISPP

Certificate issued in respect of the ship; and

(ii)	the effluent does not produce visible floating solids nor

cause discoloration of the surrounding water.

(2) For the avoidance of doubt, when the sewage is mixed with

wastes or waste water which is regulated by the other regulations under

the Ordinance, those regulations shall be complied with in addition to

this Regulation.

SECT 29

Exceptions to section 28 VerDate:02/02/2007

Section 28 does not apply if the discharge of sewage into the sea from a

ship-

(a)	is necessary for the purpose of securing the safety of the ship

and persons on board the ship or of saving life at sea;

(b)	results from damage to the ship or its equipment, provided that

all reasonable steps had been taken before and after the occurrence of the

damage for the purpose of preventing or minimizing the discharge of

sewage; or

(c)	is made-

(i)	in waters within the jurisdiction of another place; and

(ii)	in accordance with the law of that place.

SECT 30

Offences and penalties VerDate:02/02/2007

PART 6

OFFENCES AND PENALTIES

(1) If section 8, 22, 23, 24(1) or 28(1) is contravened, the

company and the master of the ship concerned each commits an offence and

is liable-

(a)	on conviction upon indictment, to a fine at level 6;

(b)	on summary conviction, to a fine at level 3.

(2) If section 25(4) or 27(3) is contravened, the company of the ship

concerned commits an offence and is liable-

(a)	on conviction upon indictment, to a fine at level 6;

(b)	on summary conviction, to a fine at level 3.

(3) If a person is charged under this section for contravening a

provision of this Regulation, it is a defence of the person to show that

he had taken all reasonable steps to ensure that the provision was

complied with.

SCHEDULE

SCHEDULE VerDate:02/02/2007

[sections 9, 10, 21,

23, 26 & 28]

REQUIREMENTS FOR EQUIPMENTS, ETC.

1.	The ship concerned shall be equipped with-

(a)	a sewage treatment plant which is in compliance with the

International Effluent Standards for Sewage Treatment Plants and the

Guidelines for Performance Tests for Sewage Treatment Plants with respect

to Effluent Standards adopted by the International Maritime Organization

by resolution MEPC.2(VI);

(b)	a sewage comminuting and disinfecting system fitted with

facilities for the temporary storage of sewage when the ship is at a

distance of not more than 3 nautical miles from the nearest land; or

(c)	a holding tank with a capacity or holding tanks with total

capacity adequate to retain all the sewage produced on board the ship in

any voyage that may be engaged by the ship until the sewage is discharged

from the ship properly in accordance with section 28(1) of this

Regulation. The holding tank shall have a means to indicate visually the

amount of its content.

2.	The ship concerned shall be equipped with a discharge manifold

for connection to reception facilities for the discharge of sewage. The

discharge manifold shall be located on the open deck of the ship with

piping. This piping shall be fitted with a flange in accordance with the

following dimensions-

Description	Dimension

Outside diameter	210 mm

Inner diameter	According to pipe outside diameter

Bolt circle diameter	170 mm

Slots in flange	4 holes, 18 mm in diameter, equidistantly placed on a bolt circle

of 170 mm diameter, slotted to the flange periphery, the slot width to be

18 mm

Flange thickness	16 mm

Bolts and nuts:

quantity and diameter	4 each of 16 mm in diameter and of suitable length

The flange shall be designed to accept pipes up to a maximum internal

diameter of 100 mm and shall be of steel or other equivalent material

having a flat face. This flange, together with a suitable gasket, shall be

suitable for a service pressure of 600 kPa.