SEA POLLUTION (PREVENTION OF POLLUTION BY GARBAGE FROM SHIPS)

REGULATIONS 1994

ARRANGEMENT OF REGULATIONS.

Regulation

1. Citation and commencement.

2. Interpretation and application.

3. Disposal of garbage outside special areas.

4. Special requirements for disposal of garbage.

5. Disposal of garbage within special areas.

6. Reception facilities.

SCHEDULE.

Form for reporting alleged inadequacy of port reception facilities

for garbage.

S.I. No. 45 of 1994.

I, DAVID ANDREWS, Minister for the Marine, in exercise of the

powers conferred on me by sections 10 of the Sea Pollution Act,

1991 (No. 27 of 1991), and for the purpose of giving effect to

the MARPOL Convention, Annex V, hereby make the following

Regulations:

REG 1

Citation and commencement.

1. These Regulations may be cited as the Sea Pollution (Prevention

of Pollution by Garbage from Ships) Regulations, 1994 and shall come

into operation on the 1st day of June, 1994.

REG 2

Interpretation and application.

2. (1) In these Regulations, except where the context otherwise

requires:

"the Act" means the Sea Pollution Act, 1991;

"garbage" means all kinds of victual, domestic and operational waste

excluding fresh fish and parts thereof, generated during the normal

operation of the ship and liable to be disposed of continuously or

periodically except those substances which are defined or listed in

other Annexes to the MARPOL Convention;

the "MARPOL Convention" means the International Convention for the

Prevention of Pollution from Ships, 1973, as amended by the Protocol

thereto, 1978;

the "Minister" means the Minister for the Marine;

"nearest land". The term "from the nearest land" means from the

baseline from which the territorial sea of the territory in question

is established in accordance with the Convention on the Territorial

Sea and the Contiguous Zone, 1958 except that, for the purposes of

the MARPOL Convention "from the nearest land off the north-eastern

coast of Australia" shall mean from a line drawn from a point on

the coast of Australia in—RD>latitude 11°00' S, longitude 142°08' E

to a point in latitude 10°35' S, longitude 141°55' E, thence to a

point latitude 10°00' S, longitude 142°00' E, thence to a point

latitude 9°10' S, longitude 143°52' E, thence to a point latitude

9°00' S, longitude 144°30' E, thence to a point latitude 13°00' S,

longitude 144°00' E, thence to a point latitude 15°00' S, longitude

146°00' E, thence to a point latitude 18°00' S, longitude 147°00'

E, thence to a point latitude 21°00' S, longitude 153°00' E, thence

to a point on the coast of Australia in latitude 24°42' S,

longitude 153°15' E;

the "Organisation" means the International Maritime Organisation;

a "Party" means a State which has ratified the MARPOL Convention;

"ship" means a vessel of any type whatsoever operating in the

marine environment and includes hydrofoil boats, aircushion vehicles,

submersibles, floating craft and fixed or floating platforms and

includes fixtures,fittings and equipment;

"special area" means a sea area where for recognised technical

reasons in relation to its oceanographical and ecological condition

and to the particular character of its traffic the adoption of

special mandatory methods for the prevention of sea pollution by

garbage is required. Special areas include those listed in Regulation

5 of these Regulations.

(2) The provisions of these Regulations shall apply to all ships to

which the Act applies.

REG 3

Disposal of garbage outside special areas.

3. (1) Subject to the provisions of Regulations 4 and 5 of these

Regulations:

( a ) the disposal into the sea of all plastics, including but

not limited to synthetic ropes, synthetic fishing nets and plastic

garbage bags, is prohibited;

( b ) the disposal into the sea of the following garbage shall be

made as far as practicable from the nearest land but is in any

case prohibited if the distance from the nearest land is less than:

(i) 25 nautical miles for dunnage, lining and packing materials

which will float;

(ii) 12 nautical miles for food wastes and all other garbage

including paper products, rags, glass, metal, bottles, crockery and

similar refuse;

( c ) disposal into the sea of garbage specified in subparagraph

(b) (ii) of this Regulation may be permitted when it has passed

through a comminuter or grinder and made as far as practicable from

the nearest land but in any case is prohibited if the distance

from the nearest land is less than 3 nautical miles. Such

comminuted or ground garbage shall be capable of passing through a

screen with openings no greater than 25 millimetres.

(2) When the garbage is mixed with other discharges having different

disposal or discharge requirements the more stringent requirements

shall apply.

REG 4

Special requirements for disposal of garbage.

4. (1) Subject to the provisions of paragraph (2) of this

Regulation, the disposal of any materials regulated by these

Regulations is prohibited from fixed or floating platforms engaged in

the exploration, exploitation and associated offshore porcessing of

seabed mineral resources, and from all other ships when alongside or

within 500 metres of such platforms.

(2) The disposal into the sea of food wastes may be permitted when

they have been passed through a comminuter or grinder from such

fixed or floating platforms located more than 12 nautical miles from

land and all other ships when alongside or within 500 metres of

such platforms. Such comminuted or ground food wastes shall be

capable of passing through a screen with openings no greater than

25 millimetres.

REG 5

Disposal of garbage within special areas.

5. (1) For the purposes of these Regulations the special areas are

the Mediterranean Sea area, the Baltic Sea area, the Black Sea

area, the Red Sea area, the "Gulfs area", the North Sea area, the

Antarctic area and the Wider Caribbean Region, including the Gulf of

Mexico and the Caribbean Sea, which are defined as follows:

( a ) The Mediterranean Sea area means the Mediterranean Sea proper

including the gulfs and seas therein with the boundary between the

Mediterranean and the Black Sea constituted by the 41°N parallel and

bounded to the west by the Straits of Gibraltar at the meridian

5°36'W;

( b ) The Baltic Sea area means the Baltic Sea proper with the

Gulf of Bothnia and the Gulf of Finland and the entrance to the

Baltic Sea bounded by the parallel of the Skaw in the Skagerrak at

57°44.8'N;

( c ) The Black Sea area means the Black Sea proper with the

boundary between the Mediterrean and the Black Sea constituted by

the parallel 41°N;

( d ) The Red Sea area means the Red Sea proper including the

Gulfs of Suez and Aqaba bounded at the south by the rhumb line

between Ras si Ane (12°8.5'N, 43°19.6'E) and Husn Murad (12°40.4'N,

43°30.2'E);

( e ) The Gulfs area means the sea area located north-west of the

rhumb line between Ras al Hadd (22°30'N, 59°48"E) and Ras al Fasteh

(25°04'N, 61°25'E);

( f ) The North Sea area means the North Sea proper including

seas therein with the boundary between:

(i) the North Sea southwards of latitude 62°N and esstwards of

longitude 4°W;

(ii) the Skagerrak, the southern limit of which is determined east

of the Skaw by lattitude 57°44.8"N; and

(iii) the English Channel and its approaches eastwards of longitude

5°W and northwards of latitude 48°30'N;

( g ) The Antarctic area means the sea area south of latitude

60°S;

( h ) The Wider Caribbean Region, as defined in article 2,

paragraph 1 of the Convention for the Protection and Development of

the Marine Environment of the Wider Caribbean Region (Cartagena de

Indias, 1983), means the Gulf of Mexico and Caribbean Sea proper

including the bays and seas therein and that portion of the

Atlantic Ocean within the boundary constituted by the 30°N parallel

from Florida eastward to 77°30'W meridian, thence a rhumb line to

the intersection of 20°N parallel and 59°W meridian, thence a rhumb

line to the intersection of 7°20'N parallel and 50°W meridian,

thence a rhumb line drawn south-westerly to the eastern boundary of

French Guiana.

(2) Subject to section 11 of the Act—

( a ) disposal into the sea of the following is prohibited:

(i) all plastics, including but not limited to synthetic ropes,

synthetic fishing nets and plastic garbage bags; and

(ii) all other garbage, including paper products, rags, glass, metal,

bottles, crockery, dunnage, lining and packing materials;

( b ) except as provided in subparagraph (c) of this paragraph,

disposal into the sea of food wastes shall be made as far as

practicable from land, but in any case not less than 12 nautical

miles from the nearest land;

( c ) disposal into the Wider Caribbean Region of food wastes

which have been passed through a comminuter or grinder shall be

made as far as practicable from land, but in any case not subject

to Regulation 4 not less than 3 nautical miles from the nearest

land. Such comminuted or ground food wastes shall be capable of

passing through a screen with openings no greater than 25

millimetres.

(3) When the garbage is mixed with other discharges having different

disposal or discharge requirements the more stringent requirements

shall apply.

(4) Reception facilities within special areas:

( a ) The Minister, in the event that any of the coastline will

border a special area, undertakes to ensure that as soon as

possible in all ports within the special area, adequate reception

facilities are provided in accordance with Regulation 6 of these

Regulations, taking into account the special needs of ships operating

in these areas.

( b ) The Minister shall notify the Organisation of the measures

taken pursuant to subparagraph (a) of this Regulation. Upon receipt

of sufficient notifications the Organisation shall establish a date

from which the requirements of this Regulation in respect of the

area in question shall take effect. The Organisation shall notify

all Parties of the date so established no less than twelve months

in advance of that date.

( c ) After the date so established, ships calling also at ports

in these special areas where such facilities are not yet available,

shall fully comply with the requirements of this Regulation.

(5) The following rules apply in relation to the Antarctic area:

( a ) Where, at a port in the State, a ship departs to or

arrives from the Antarctic area the Minister shall ensure that as

soon as practicable adequate facilities are provided for the

reception of all garbage from the ship without causing undue delay,

and according to the needs of the ships using them.

( b ) The Minister shall ensure that all ships entitled to fly

the Irish flag,before entering the Antarctic area, have sufficient

capacity on board for the retention of all garbage while operating

in the area and have concluded arrangements to discharge such

garbage at a reception facility after leaving the area.

REG 6

Reception facilities.

6. (1) Harbour authorities and persons having control of a harbour

shall provide facilities at ports and terminals for the reception of

garbage, without causing undue delay to ships, and according to the

needs of the ships using them.

(2) The Minister shall notify the Organisation for transmission to

the Parties concerned of all cases where the facilities provided

under this Regulation are alleged to be inadequate.

(3) The form for reporting alleged inadequacy shall be drawn up in

the form set out in the Schedule to these Regulations.

SCHEDULE.

FORM FOR REPORTING ALLEGED INADEQUACY OF PORT RECEPTION FACILITIES

FOR GARBAGE.

1. Country

Name of port or area

................................................................................................................................Location

in the port (e.g. berth/terminal/jetty)

....................................................................................Date

of incident

........................................................................................................................................

2. Type and amount of garbage for discharge to facility

(a) Total amount (m3)

food waste

..........................................................................................................................................cargo-associated

waste

....................................................................................................................maintenance

waste

............................................................................................................................other

....................................................................................................................................................

(b) Amount not accepted by the facility:

food waste

...........................................................................................................................................cargo-associated

waste

.....................................................................................................................maintenance

waste

.............................................................................................................................other

......................................................................................................................................................

3. Special problems encountered (Tick as appropriate and give

details):

Undue delay Yes/No. _____________________________________________________

Inconvenient location of facilities Yes/No.

_____________________________________

______________________________________________________________________

Unreasonable charges for use of facilities Yes/No.

________________________________

_______________________________________________________________________

Use of facility not technically possible Yes/No.

___________________________________

_______________________________________________________________________

Special national regulations Yes/No.

___________________________________________

_______________________________________________________________________

Other _________________________________________________________________

4. Remarks: (e.g. information received from port authorities or

operators of reception facilities, reasons given concerning 2(b)

above)

_______________________________________________________________________

_______________________________________________________________________

_______________________________________________________________________

5. Ship's particulars:

Name of ship

..............................................................................................................................

Owner or operator

.....................................................................................................................

Distinctive number or letters

...................................................................................................

Port of registry

...........................................................................................................................

Number of persons on board

...................................................................................................

..........................................................................................................................................................Date

of completion of form.Signature of Master.

GIVEN under my Official Seal, this 18th day of February, 1994.

DAVID ANDREWS,

Minister for the Marine.

EXPLANATORY NOTE.

These Regulations give effect to Annex V of the International

Convention for the Prevention of Pollution from Ships, adopted by

the International Maritime Organisation on 2 November, 1973 and as

amended by its Protocol adopted on 17 February, 1978, and as

further amended by the Marine Environment Protection Committee (MEPC)

of the International Maritime Organisation under Reslutions which

designated the North Sea, the Antarctic area (south of latitude

60°S) and the Wider Caribbean Region as "special areas".

The Regulations apply to all Irish ships wherever they may be and

to all other ships when they are in the territorial seas and

inland waters of the State.

The Regulations prohibit and control the disposal of garbage into

the sea in accordance with the type of garbage for disposal and

the geographical location of the ship. They also provide for the

availability of adequate facilities at ports and terminals for the

reception of garbage.

Section 29 of the Sea Pollution Act, 1991 provides for penalties

for breaches of these Regulations.