Reprint as at 9 June 2005

Hazardous Substances (Minimum Degrees of Hazard) Regulations 2001

(SR 2001/112)

Silvia Cartwright, Governor-General

Order in Council

At Wellington this 28th day of May 2001

Present:

Her Excellency the Governor-General in Council

Pursuant to sections 74(b) and 75(1)(g) of the Hazardous Substances and New

Organisms Act 1996, Her Excellency the Governor-General, acting on the

advice and with the consent of the Executive Council, makes the following

regulations (given on the recommendation of the Minister for the Environment

made in accordance with section 141(1) of that Act).

Contents

1   Title

2   Commencement

3   Interpretation

4   Substances not considered hazardous

5   Medicines

6   Food

7   Minimum degrees of hazard

Schedule 1

Minimum degrees of hazard for substances with explosive properties

Schedule 2

Minimum degrees of hazard for substances with flammable properties

Schedule 3

Minimum degrees of hazard for substances with oxidising properties

Schedule 4

Minimum degrees of hazard for substances with toxic properties

Schedule 5

Minimum degrees of hazard for substances with corrosive properties

Schedule 6

Minimum degrees of hazard for substances with ecotoxic properties

Regulations

1 Title

These regulations are the Hazardous Substances (Minimum Degrees of Hazard)

Regulations 2001.

2 Commencement

These regulations come into force on 2 July 2001.

3 Interpretation

In these regulations, unless the context otherwise requires,---

Act   means the Hazardous Substances and New Organisms Act 1996

ASTM, when followed by letters and numbers, means the document identified by those letters and numbers that is published by the American Society for Testing and Materials

data   includes values that are directly measured, calculated, or estimated for any of the measures given

deflagrate, in relation to a substance that is initiated or ignited, means the production in that substance of a chemical reaction that proceeds through, or along the surface of, the substance at subsonic velocity, where that chemical reaction,---

(a)  results in the steady production of hot gases at high pressures; and

(b)  if the substance is sufficiently confined, results in an increase in pressure, rate of reaction, and temperature that may produce a detonation of the substance

detonate, in relation to a substance that is initiated, means the production in that substance of a chemical reaction that proceeds through that substance at supersonic velocity, resulting in the production of heat and a supersonic shock wave through the surrounding medium

gas   means a substance that---

(a)  is completely gaseous at 20oC and at 101.3 kPa absolute pressure; or

(b)  has a vapour pressure of more than 300 kPa absolute pressure at 50oC

ISO   means the International Standards Organisation; and---

(a)  ISO 9328 (II): 1991  means the ISO Standard called Steel plates and strips for pressure purposes---technical delivery conditions, Part II; and

(b)  ISO 10156: 1996 means the ISO Standard called Gases and gas

mixtures---determination of fire potential and oxidising ability for the

selection of cylinder valve outlets

liquid   means---

(a)  a substance with a melting point of less than or equal to 20oC at 101.3 kPa absolute pressure; or

(b)  a viscous substance, without a defined melting point, if---

(i)     more than the quantity of the substance specified in ASTM D 4359-90, called Test method for determining whether a material is a liquid or a solid, collects on a watch glass when tested in the manner specified in that test; or

(ii)   a penetrometer penetrates into the substance the distance

defined in the test for determining fluidity prescribed in Appendix A.3 of the European Agreement concerning the International Carriage of Dangerous Goods by Road (ADR), published in 1994 by the United Nations, when the method specified in that test is followed

pyrotechnic effect, in relation to a substance that is initiated, means the

production in that substance of a self-sustaining exothermic chemical reaction resulting in heat, sound, light, gas, smoke, or motion, or a combination of these

solid   means a substance that is neither a liquid nor a gas

Test Series, when followed by a letter or number, means 1 or more tests as

prescribed in the UN Manual of Tests and Criteria

UN Manual of Tests and Criteria   means the third revised edition of the

Recommendations on the Transport of Dangerous Goods Manual of Tests and

Criteria, published in 1999 by the United Nations (New York and Geneva)

UN Model Regulations  means the 11th revised edition of the Recommendations

on the Transport of Dangerous Goods Model Regulations, published in 1999 by the United Nations (New York and Geneva).

4 Substances not considered hazardous

(1)  A substance is not hazardous for the purposes of the Act unless it meets the minimum degrees of hazard for at least 1 of the intrinsic hazardous substance properties specified in regulation 7.

(2)  This regulation is subject to regulations 5 and 6.

5 Medicines

(1)  A medicine is not hazardous for the purposes of the Act unless Parts 13, 14, or 15 of the Act apply.

(2)  Despite subclause (1), a new medicine must be treated as hazardous if it meets any of the minimum degrees of hazard prescribed in these regulations and---

(a)  it is a substance to which section 3(1)(b) of the Medicines Act 1981 applies; or

(b) an application is made to register that medicine as a trade name

product under the Agricultural Compounds and Veterinary Medicines Act 1997.

(3)  In this regulation,---

medicine   has the same meaning as in section 3(1) of the Medicines Act 1981, except that it does not include a gas contained at a pressure greater than 170 kPa in a container larger than 100 mL, at any time after that gas becomes so contained and before the time the gas is first administered to a person for a therapeutic purpose

new medicine   means---

(a)  any medicine that has not been generally available in New Zealand---

(i)     before the commencement of the Medicines Act 1981; or

(ii)    at any time during the period of 5 years immediately before the date on which it is proposed to become so available:

(b)  any medicine that, immediately before the commencement of Part 2 of the Medicines Act 1981, was a therapeutic drug to which section 12 of the Food and Drug Act 1969 applied, and in respect of the sale or distribution of which the Minister of Health had not given his or her consent under that section:

(c)  any medicine that becomes a medicine for the first time after the commencement of the Medicines Act 1981:

(d)  any medicine that is referred to the Minister of Health under

section 24(5) of the Medicines Act 1981.

6 Food

(1)  A food is not hazardous for the purposes of the Act unless Parts 13, 14, or 15 of the Act apply.

(2)  In this regulation,---

food   has the same meaning as in section 2 of the Food Act 1981, except that it does not include a food additive if that food additive has not been mixed with or added to any other food or drink

food additive   has the same meaning as in section 2(1) of the Food Regulations 1984 (SR 1984/262).

7 Minimum degrees of hazard

(1)  The minimum degrees of hazard for substances with explosive properties are the degrees of hazard specified in Schedule 1.

(2)  The minimum degrees of hazard for substances with flammable properties are the degrees of hazard specified in Schedule 2.

(3)  The minimum degrees of hazard for substances with oxidising properties are the degrees of hazard specified in Schedule 3.

(4)  The minimum degrees of hazard for substances with toxic properties are the degrees of hazard specified in Schedule 4.

(5)  The minimum degrees of hazard for substances with corrosive properties are the degrees of hazard specified in Schedule 5.

(6)  The minimum degrees of hazard for substances with ecotoxic properties are the degrees of hazard specified in Schedule 6.

Schedule 1                            r 7(1)

Minimum degrees of hazard for substances with explosive properties

1  Minimum degrees of hazard for substances other than manufactured articles

A substance (other than a manufactured article) with explosive properties

is not hazardous for the purposes of the Act unless---

(a) the substance is listed as class 1 in the Dangerous Goods List in Chapter 3.2 of the UN Model Regulations; or

(b) the substance propagates a detonation in accordance with paragraph 12.4.1.4     of the UN Manual of Tests and Criteria when tested as prescribed by Test Series 2 type&nbsp;(a) in paragraph 12.4 of that manual; or

(c) the substance propagates a detonation in accordance with paragraph 12.5.1.4  of the UN Manual of Tests and Criteria when tested as prescribed by Test Series 2 type&nbsp;(b) in paragraph 12.5 of that manual; or

(d) the substance produces a pressure rise from 690 to 2070 kPa gauge pressure or more, as determined in accordance with paragraph 12.6.1.4 of the UN Manual of Tests and Criteria, when tested as prescribed by Test Series 2 type (c) in paragraph 12.6 of that manual; or

(e) the substance is specifically manufactured to detonate, deflagrate, or

produce a pyrotechnic effect.

2       Minimum degrees of hazard for manufactured articles

A manufactured article containing, incorporating, or including a hazardous

substance with explosive properties is not hazardous for the purposes of the Act unless---

(a)  the article is listed as class 1 in the Dangerous Goods List in

Chapter 3.2 of the UN Model Regulations; or

(b)  the article produces some effect of projection, fire, smoke, heat, or loud noise external to the article when tested as a stack of articles as

prescribed in Test Series 6 type (c) in paragraph 16.6 of the UN Manual of Tests and Criteria.

Schedule 2                    r 7(2)

Minimum degrees of hazard for substances with flammable properties

1 Interpretation

In this schedule, unless the context otherwise requires,---

flammable ingredient   means a substance that meets 1 or more of the minimum degrees of hazard for a flammable gas, a flammable liquid, or a flammable solid, or any combination of them

flammable range---

(a)  means the range between 2 ratios of gas or vapour to air, the lower of which contains too much air, and the upper of which contains too little air, to support combustion; and

(b)  includes a minimal range effectively equivalent to a single value

flashpoint   means the lowest temperature at which a flammable liquid, when

tested in a closed cup flash point test, gives off vapour that ignites

SADT   means self-accelerating decomposition temperature, being the lowest

temperature at which self-accelerating decomposition of the substance occurs in the packaging in which it is tested when tested as prescribed in Test Series H in section 28 of the UN Manual of Tests and Criteria.

2 Minimum degrees of hazard

(1)  A substance with flammable properties is not hazardous for the purposes of the Act unless---

(a)  the substance is a gas or a gas mixture that is sufficiently

flammable to be capable of ignition when mixed with air in a proportion within a flammable range at 20oC and at a pressure of 101.3 kPa absolute; or

(b)  the substance---

(i)     is or contains a gas compressed, liquefied, or dissolved

under pressure (with or without a liquid, paste, or powder); and

(ii)    is packed under pressure in a way that is designed to be

released---

(A)  as solid or liquid particles in suspension in a gas; or

(B)  as a foam, paste, or powder; or

(C)  in a liquid state; or

(D)  in a gaseous state; and

(iii)   comprises 45% or more by mass of flammable ingredients; or

(c)  the substance is a liquid that has a flash point of less than or

equal to 93oC; or

(d)  the substance---

(i)     is described in paragraph 2.4.2.4 of the UN Model Regulations as being an explosive substance that has been dissolved or suspended in water or other liquid substances to form a homogeneous mixture in order to suppress its explosive properties; or

(ii)    is listed in paragraph 2.3.1.4 of the UN Model Regulations with 1 of the serial numbers UN 1204, UN 2059, UN 3064, or UN 3343; or

(iii)   is listed in the Dangerous Goods List in Chapter 3.2 of the UN Model Regulations as having a class, division, or subsidiary risk of liquid desensitised explosive; or

(e)  the substance is a solid that meets the criteria specified in

paragraph 33.2.1.4.4 of the UN Manual of Tests and Criteria when tested in

accordance with the test method for readily combustible solids as prescribed in Test Series N.1 in paragraph 33.2.1.4 of that manual; or

(f)  the substance is a solid that---

(i)     is listed in the Dangerous Goods List in Chapter 3.2 of the UN Model Regulations; and

(ii)    has 1 of the serial numbers UN 1331, UN 1343, UN 1944, UN

1945, or UN 2254; or

(g)  the substance---

(i)     has an SADT of less than or equal to 75oC in a quantity of 50 kg and a heat of decomposition greater than 300 joules per gram; or

(ii)    is listed in paragraph 2.4.2.3.2.3 of the UN Model

Regulations as having a class, division, or subsidiary risk of self-reactive; or

(h)  the substance---

(i)     meets 1 or more of the minimum degrees of hazard for

substances with explosive properties specified in Schedule 1; and

(ii)    has been desensitised to the extent that it would, when

tested as prescribed in Test Series 6 type (c) in paragraph 16.6 of the UN

Manual of Tests and Criteria, not show a projection, fire, smoke, heat, or

noise effect external to the substance itself; or

(i)  the substance---

(i)     is described in paragraph 2.4.2.4 of the UN Model

Regulations as being an explosive substance that has been wetted with water or alcohols or diluted with other substances to form a homogeneous mixture in order to suppress its explosive properties; or

(ii)    is listed in paragraph 2.4.2.4 of the UN Model Regulations; or

(iii)   is listed in the Dangerous Goods List in Chapter 3.2 of the UN Model Regulations as having a class, division, or subsidiary risk of a solid desensitised explosive; or

(j)  the substance---

(i)     is a solid in powder form that, when tested as prescribed in Test Series N.2 in paragraph 33.3.1.4 of the UN Manual of Tests and Criteria, ignites in 1 of the tests; or

(ii)    is a liquid that, when tested in as prescribed in Test Series N.3 in paragraph 33.3.1.5 of the UN Manual of Tests and Criteria, ignites in the first part of the test (paragraph 33.3.1.5.3.1) or ignites or chars the filter paper in the second part of the test (paragraph 33.3.1.5.3.2); or

(iii)   is a solid that, when tested as prescribed in Test Series N.4 in paragraph 33.3.1.6 of the UN Manual of Tests and Criteria, gives a positive result in a test using a 100 mm sample cube at 140oC; or

(k)  the substance, when tested as prescribed in Test Series N.5 in

paragraph 33.4.1.4 of the UN Manual of Tests and Criteria, reacts with water at ambient temperatures to produce a gas that ignites spontaneously; or

(l)  the substance, when tested as prescribed in Test Series N.5 in

paragraph 33.4.1.4 of the UN Manual of Tests and Criteria, reacts with water at ambient temperatures to produce a flammable gas at a rate greater than or equal to 1 litre per kilogram of substance per hour.

(2)  When tested in accordance with any of subclause (1)(e), (g), (h),

(j)(i), (j)(iii), (k), or (l), a substance must be---

(a)  in the finest particle form in which that substance is reasonably capable of being used or rendered; or

(b)  if it is likely or known that more than 10% of the mass of the

substance will crumble into a finer particle form, in that finer form.

Schedule 3                           r 7(3)

Minimum degrees of hazard for substances with oxidising properties

1 Interpretation

In this schedule, unless the context otherwise requires,---

organic peroxide   means a substance containing 1 or more chemical compounds that---

(a)  contains the bivalent oxygen [-0-0-] structure; and

(b)  may be considered as a derivative of hydrogen peroxide where 1 or both of the hydrogen atoms has been replaced by an organic radical; and

(c)  may cause or contribute to combustion by the release of chemical

energy or compounds that may cause or contribute to fire, explosion, or

chemical decomposition

oxidising substance   means a substance that, while not necessarily combustible in itself, may cause or contribute to the combustion of other substances or materials.

2 Minimum degrees of hazard

(1)  A substance with oxidising properties is not hazardous for the purposes of the Act unless---

(a)  the substance is listed in the Dangerous Goods List in Chapter 3.2 of the UN Model Regulations as having a class, division, or subsidiary risk of 5.1 (oxidising substances); or

(b)  the substance is a solid that---

(i)     is an oxidising substance; and

(ii)    when mixed with dried cellulose, either spontaneously ignites or shows a mean burning time equal to or faster than the mean burning time of a 3:7 reference mixture by mass of potassium bromate and cellulose under the same conditions when the mixture is tested in accordance with the test method for

oxidising solids as prescribed in Test Series O.1 in paragraph 34.4.1 of the UN Manual of Tests and Criteria; or

(c)  the substance is a liquid that---

(i)     is an oxidising substance; and

(ii)    when mixed with dried cellulose, either spontaneously ignites or shows a mean pressure rise time that is equal to or faster than the mean pressure rise time of the reference mixture of 65% aqueous nitric acid solution and cellulose under the same conditions when that solution is tested in accordance with the test method for oxidising liquids as prescribed in Test Series O.2 in paragraph 34.4.2 of the UN Manual of Tests and Criteria; or

(d)  the substance is a gas that---

(i)     is an oxidising substance; and

(ii)    will cause or contribute to combustion at a faster rate than air when tested in accordance with the test method for determining the

oxidising power of gases and gas mixtures as prescribed in ISO 10156:1996; or

(e)  the substance is listed in---

(i)     paragraph 2.5.3.2.4 of the UN Model Regulations as an organic peroxide; or

(ii)    the Dangerous Goods List in Chapter 3.2 of the UN Model

Regulations as having a class or division of 5.2 (organic peroxides); or

(f)  the substance---

(i)     is an organic peroxide or contains organic peroxides; and

(ii)    has more than 1% available oxygen from the organic peroxides when containing not more  than 1% hydrogen peroxide by mass; or

(g)  the substance---

(i)     is an organic peroxide or contains organic peroxides; and

(ii)    has more than 0.5% available oxygen from the organic

peroxides when containing not less than 1% and not more than 7% hydrogen

peroxide by mass.

(2)  If a substance is a mixture and is made up of 1 or more chemical

elements or compounds, any 1 of which meets 1 or more of the minimum degrees of hazard specified in subclause (1)(b), (c), (f), or (g), then the mixture is hazardous for the purposes of the Act unless it can be shown that the exact mixture itself does not meet any of the minimum degrees of hazard specified in subclause (1).

(3)  When tested in accordance with subclause (1)(b), a substance must be---

(a)  in the finest particle form in which that substance is reasonably capable of being used or rendered; or

(b)  if it is likely or known that more than 10% of the mass of the

substance will crumble into a finer particle form, in that finer form.

(4)  For the purposes of subclause (1)(f) and (g), the available oxygen

content as a percentage by mass must be determined in accordance with the

following formula:

O% = 16 SUM ( n  [ c/m ] )

i      i

where---

O%           is the percentage of available oxygen content to be determined

SUM          is the symbol for summation where there is more than 1

organic peroxide

n            is the number of peroxygen groups per molecule of each organic

i           peroxide

C            is the percentage concentration by mass of each organic peroxide

i

m            is the molecular mass of each organic peroxide.

i

Schedule 4                 r 7(4)

Minimum degrees of hazard for substances with toxic properties

1 Interpretation

In this schedule, unless the context otherwise requires,---

developmental effect, in relation to an organism, includes structural

abnormality, altered growth, functional deficiency, or interference with the normal development of an organism (including the death of a developing

organism), that is---

(a)  manifested at any point in the organism's life span; and

(b)  caused by---

(i)     the exposure of a parent to the substance before conception; or

(ii)    the exposure of the developing offspring to the substance

during prenatal development or postnatal development up to the time of sexual maturation

dust and mist, in relation to a substance in the atmosphere, means that 90% of the substance is in the form of particles with a aerodynamic diameter of less than 10 microns

expert   means---

(a)  a member of a scientific committee set up by an international,

national, or professional scientific body to review scientific data; or

(b)  a person considered by his or her scientific peers to be an expert in the relevant field of scientific study

genotoxic effect   means alterations to the structure, information content, or segregation of DNA, including---

(a)  DNA damage caused by interference with its normal replication

processes; and

(b)  temporary non-physiological alterations to its replication

LD  50   means the median lethal dose, being a statistically derived single dose of a substance that can be expected to cause death in 50% of animals

mean Draize score,---

(a)  in relation to acute skin irritation tests, means the mean value in at least 2 of 3 tested animals---

(i)     from Draize grades measured at intervals of 24 hours, 48

hours, and 72 hours after the patch is removed; or

(ii)    where reactions are delayed, from Draize grades on 3

consecutive days after the onset of dermal reactions; and

(b)  in relation to acute eye irritation tests, means the mean value of at least 2 of 3 tested animals from Draize grades measured at intervals of 24 hours, 48 hours, and 72 hours after instillation of the substance

mutagenic effect   means a permanent change in the amount or structure of the genetic material in a cell, being a permanent change that is---

(a)  manifested at the phenotypic level; or

(b)  an underlying DNA modification (including specific base pair changes and chromosomal translocations)

reliable information   means information that is derived from---

(a)  a valid and relevant animal study conducted in accordance with

internationally accepted test guidelines and principles of good laboratory

practice; or

(b)  an epidemiological study in humans that is statistically sound and has undergone peer review; or

(c)  any other study whose relevance and validity can be demonstrated

according to internationally accepted criteria and scientific practice

reproductive effect   includes---

(a)  interference with reproductive ability or capacity, including

alteration to the male or female reproductive system; or

(b)  an effect on the onset of puberty, gamete production and transport, reproductive cycle normality, sexual behaviour, fertility, parturition, or premature reproductive senescence; or

(c)  an effect on or through lactation; or

(d)  modifications in other functions that are dependent on the integrity of the reproductive system

sensitisation   means an immunologically mediated response where, after

exposure to a substance to which an organism or human being has been previously exposed, the organism or human being is, or 1 or more organs in an organism or human being are, more readily and adversely affected by that substance

significant adverse biological effect   means a toxicologically significant

change in an organ or in an animal observed during the study where the

probability that the change is different from any recognised background history of change or from the value in a recognised unexposed control organ or animal group in the test animal strain is greater than 0.95 (equivalent to P (probability) of 0.05 or less)

valid, in relation to a study, means---

(a)  the design of the study methodology accurately reflects the matters the study seeks to measure; and

(b)  the study findings can be extrapolated from the sample used in the study to a broader population.

2 Minimum degrees of hazard

(1)  A substance with toxic properties is not hazardous for the purposes of the Act unless---

(a)  data for the substance indicates an LD<inf>50 of 5 000 milligrams or less of the substance per kilogram of bodyweight as a result of acute exposure of animals to the substance by oral or dermal routes; or

(b)  data for the substance indicate any mortality, as a result of acute exposure of animals by---

(i)     oral or dermal routes to 2 000 milligrams or less of the

substance per kilogram of bodyweight; or

(ii)    the inhalation route to---

(A)     5 000 parts or less of the substance per million in air, if the substance is a gas; or

(B)     20 milligrams or less of the substance per litre of air, if the substance is a vapour; or

(C)     5 milligrams or less of the substance per litre of air, if the substance is a dust or mist; or

(c)  clinical signs (other than diarrhoea, piloerection, or an ungroomed appearance) indicate to an expert a significant adverse biological effect as a result of acute exposure of animals by---

(i)     oral or dermal routes to 2 000 milligrams or less of the

substance per kilogram of bodyweight; or

(ii)    the inhalation route to---

(A)     5 000 parts or less of the substance per million in air, if the substance is a gas; or

(B)     20 milligrams or less of the substance per litre of air, if the substance is a vapour; or

(C)     5 milligrams or less of the substance per litre of air, if the substance is a dust or mist; or

(d)  reliable information for the substance, including reliable

information from animal studies other than those from which LD                                                           50 data was obtained, where exposure was by a route other than oral, dermal, or inhalation, indicates to an expert the potential for significant acute toxic effects in humans after exposure to the substance; or

(e)  data for the substance, in the opinion of an expert, indicates

evidence in humans of significant acute toxic effects as a result of exposure to the substance; or

(f)  data for the substance indicates a mean Draize score of 1.5 or more for either of the skin irritation effects known as erythema or oedema, as a result of exposure to the substance; or

(g)  data for the substance indicates a mean Draize score of 1 or more for either of the eye irritation effects known as corneal opacity or iritis, as a result of exposure to the substance; or

(h)  data for the substance indicates a mean Draize score of 2 or more for either of the eye irritation effects known as conjunctival redness or chemosis, as a result of exposure to the substance; or

(i)     data for the substance indicates positive evidence of respiratory sensitisation in animals as a result of exposure to the substance; or

(j)  data for the substance indicates positive evidence of sensitisation by skin contact in animals as a result of exposure to the substance of either---

(i)     30% or more sensitisation response in an adjuvant type test method; or

(ii)    15% or more sensitisation response in a non-adjuvant type

test method; or

(k)  data for the substance, in the opinion of an expert, indicates

evidence in humans of specific respiratory hypersensitivity (including asthma, rhinitis, and alveolitis) as a result of exposure to the substance; or

(l)  data for the substance, in the opinion of an expert, indicates

evidence in humans of sensitisation by skin contact as a result of exposure to the substance; or

(m)  data for the substance indicates evidence of mutagenic effects as a result of mammalian in vivo exposure to the substance; or

(n)  data for the substance indicates evidence of---

(i)     genotoxic effects as a result of mammalian in vivo exposure to the substance; and

(ii)    mutagenic effects as a result of in vitro exposure to the

substance; or

(o)  data for the substance indicates evidence of mutagenic effects as a result of in vitro exposure of mammalian cells to the substance and the substance has a structure-activity relationship to known germ cell mutagens, where---

(i)     structure-activity relationship means a significant

correlative relationship between the chemical structure of the substance

and the chemical structure of a known germ cell mutagen; and

(ii)    the relationship relates to that germ cell mutagen activity; or

(p)  reliable information for the substance indicates to an expert that exposure to the substance causes the development of cancer or an increase in the incidence of benign or malignant tumours in an organ or an organism; or

(q)  reliable information for the substance indicates to an expert that exposure to the substance causes an adverse reproductive effect; or

(r)  reliable information for the substance indicates to an expert that exposure to the substance causes an adverse developmental effect; or

(s)  data for the substance indicates, in the opinion of an expert,

evidence of a significant adverse biological effect or a significant toxic

effect (other than an effect referred to in any of paragraphs (a) to (r)) on the function or morphology of an organ, or on the biochemistry or haematology of an organism or human being as a result of exposure to the substance and, in the case of a significant adverse biological effect, the change is relevant to human health.

(2)  A substance is not required to be tested in accordance with subclause

(1)(a) if the substance---

(a)  has been tested in accordance with subclause (1)(b); and

(b)  does not meet the minimum degree of hazard specified in subclause

(1)(b).

Schedule 5                    r 7(5)

Minimum degrees of hazard for substances with corrosive properties

1 Interpretation

In this schedule, unless the context otherwise requires, mean Draize score,

in relation to acute eye irritation tests, means the mean value in at least 2 of 3 tested animals from Draize grades measured at intervals of 24 hours, 48 hours, and 72 hours after instillation of the substance.

2 Minimum degrees of hazard

A substance with corrosive properties is not hazardous for the purposes of the Act unless---

(a)  the substance corrodes, at a rate exceeding 6.25 millimetres per

year at a test temperature of 55oC,---

(i)     steel type P235 (ISO 9328 (II):1991); or

(ii)    steel type SAE 1020 (Society of Automotive Engineers); or

(iii)   non-clad aluminium types SAE 7075-T6 or AZ5GU-T6; or

(b)  data for the substance indicates that the substance has a pH level of 2 or less, or 11.5 or more; or

(c)  data for the substance indicates destruction of dermal tissue, being visible necrosis through the epidermis and into the dermis, as a result of exposure to the substance, that has not fully reversed within an observation period of 14 days; or

(d)  data for the substance indicates destruction of ocular tissue, being adverse effects on the cornea, iris, or conjunctiva, as a result of exposure to the substance, that has not fully reversed within an observation period of 21 days; or

(e)  data for the substance indicates a mean Draize score of 3 or more for the eye irritation effect known as corneal opacity, as a result of exposure to the substance; or

(f)  data for the substance indicates a mean Draize score of 1.5 or more for the eye irritation effect known as iritis, as a result of exposure to the substance.

Schedule 6          r 7(6)

Minimum degrees of hazard for substances with ecotoxic properties

1 Interpretation

In this schedule, unless the context otherwise requires,---

BCF   means bioconcentration factor, being the steady state concentration of a substance in an aquatic organism divided by the concentration of the substance

in the surrounding water

bioaccumulative, in relation to a substance, means the substance has a BCF

greater than or equal to 500 or, if

BCF data is not available, a log K<inf>ow greater than or equal to 4;

and, for the purposes of this definition, measured log K<inf>ow values

take precedence over estimated values

biocidal action, in relation to a substance, means the substance causes

mortality, inhibited growth, or inhibited reproduction in an organism

BOD   means 5-day biochemical oxygen demand, being the mass of oxygen

5  consumed by micro-organisms during oxidation of the substance in water over a period of 5 days, expressed in units of milligrams of oxygen consumed per milligram of the substance

COD   means chemical oxygen demand, being the equivalent mass of oxygen from an oxidising agent, of a strength at least equal to the oxidising strength of potassium permanganate or potassium dichromate, that is consumed during oxidation of the substance in water, expressed in units of milligrams of oxygen consumed per milligram of the substance

EC    means the median effect concentration, being a statistically derived

50  concentration of a substance that can be expected to cause---

(a)  an adverse effect in 50% of organisms; or

(b)  a 50% reduction in growth or in the growth rate of organisms

K     means the steady state ratio of the solubility of a substance in

ow   n-octanol to the solubility of that substance in water

LC    means the median lethal concentration, being a statistically

50  derived concentration of a substance that can be expected to cause death in 50% of organisms exposed for a specified time

LD    means a median lethal dose, being a statistically derived single

50  dose of a substance that can be expected to cause death in 50% of

organisms

LOEC   means the lowest observed effect concentration, being the lowest

concentration of a substance that produces a significant ecotoxic effect in an organism or in an organism population

MATC   means the maximum acceptable toxicant concentration, being the geometric mean of the NOEC and LOEC where the NOEC and LOEC are derived from the same study

NOEC   means the no observed effect concentration, being the highest

concentration of a substance that does not produce a significant ecotoxic

effect in an organism or in an organism population

rapidly degradable, in relation to a substance in water, means that---

(a)  28 days after a solution containing the substance is inoculated with micro-organisms, there is at least---

(i)     a 70% reduction in dissolved organic carbon in the solution; or

(ii)    a 60% depletion of oxygen in the solution, when compared with the maximum depletion of oxygen that would occur if the substance were

completely degraded; or

(iii)   a 60% generation of carbon dioxide in the solution, when

compared with the maximum generation of carbon dioxide that would occur if the substance were completely degraded; or

(b)  if only COD and BOD<inf>5 data is available, the ratio of BOD<inf>5 to COD is greater than or equal to 0.5:1; or

(c)  at least 70% of the substance can be degraded biotically or

abiotically, in the aquatic environment within 28 days

significant ecotoxic effect   means an ecotoxicologically significant change in an organism or in an organism population observed during the study where the probability that the change is different from any recognised background history of change or from the value in a recognised unexposed control organism or organism population is greater than 0.95 (equivalent to P (probability) of 0.05 or less)

2 Minimum degrees of hazard

(1)  A substance with ecotoxic properties is not hazardous for the purposes of the Act unless---

(a)  the substance is ecotoxic to aquatic organisms because---

(i)     data for the substance indicates that the fish LC

50 is

100 milligrams or less of the substance per litre of water over a 96-hour

exposure period, as a result of exposure to the substance; or

(ii)   data for the substance indicates that the crustacean EC

50 is 100 milligrams or less of the substance per litre of water over a 48-hour exposure period, as a result of exposure to the substance; or

(iii)   data for the substance indicates that the algal or other

aquatic plant EC50 is 100 milligrams or less of the substance per litre of

water over a 72-hour or 96-hour exposure period, as a result of exposure to

the substance; or

(iv)    data for the substance indicates that the chronic fish NOEC, or chronic crustacean NOEC, or algal or other aquatic plant chronic NOEC, is 1 milligram or less of the substance per litre of water, as a result of exposure to the substance; or

(v)    in the absence of the NOEC data prescribed in subparagraph

(iv) data for the substance indicates that it is not rapidly degradable and is bioaccumulative; or

(b)  the substance is ecotoxic to soil organisms because---

(i)     data for the substance indicates that a plant or soil

invertebrate EC   50 is 100 milligrams or less of the substance per kilogram of dry weight of soil over a 14-day exposure period, as a result of exposure to the substance; or

(ii)    data for the substance indicates a 25% reduction in microbial respiration or microbial nitrification at 100 milligrams or less of the substance per kilogram of dry weight of soil after a 28-day exposure period, as a result of exposure to the substance; or

(c)  the substance is ecotoxic to terrestrial vertebrates because---

(i)     data for the substance indicates an acute avian or mammalian oral or dermal LD

50 of 2 000 milligrams or less of the substance per kilogram of body weight, as a result of exposure to the substance; or

(ii)    data for the substance indicates an acute avian or mammalian LC

50 of 5 000 parts or less of the substance per million in the diet, as a

result of exposure to the substance; or

(iii)   data for the substance indicates a chronic avian or mammalian MATC of 100 parts or less of the substance per million in the diet, as a result of exposure to the substance; or

(d)  the substance is ecotoxic to terrestrial invertebrates because data for the substance indicates an acute oral or contact LD

50 of 25 micrograms or less of the substance per terrestrial invertebrate, as a result of exposure to the substance; or

(e)  the substance is designed for biocidal action.

(2)  A substance referred to in subclause (1)(e) is not hazardous for the

purposes of this schedule if---

(a)  the substance is designed for biocidal action against a virus,

protozoan, bacterium, or an internal organism in humans or in other

vertebrates; and

(b)  the substance does not meet any of the minimum degrees of hazard

specified in subclause (1)(a) to (d).

Martin Bell,

for Clerk of the Executive Council.

Issued under the authority of the Acts and Regulations Publication Act 1989.

Date of notification in Gazette: 31 May 2001.

Index to notes

1           General

2           Status of reprints

3           How reprints are prepared

4           Changes made under section 17C of the Acts and Regulations

Publication Act 1989

5           List of amendments incorporated in this reprint (most recent

first)

Notes

1           General

This is a reprint of the Hazardous Substances (Minimum Degrees of Hazard)

Regulations 2001. The reprint incorporates all the amendments to the Hazardous Substances (Minimum Degrees of Hazard) Regulations 2001 as at 9 June 2005, as specified in the list of amendments at the end of this reprint.

2           Status of reprints

Under section 29A of the Evidence Act 1908, reprints are presumed to correctly state, as at the date of the reprint, the law enacted by the principal enactment and by the amendments to that enactment. This presumption applies even though editorial changes authorised by section 17C of the Acts and Regulations Publication Act 1989 have been made in the reprint. For an outline of editorial changes made in reprints, see below.

This presumption may be rebutted by producing the official volumes of statutes or statutory regulations in which the principal enactment and its amendments are contained.

3           How reprints are prepared

The following conventions are followed in the preparation of reprints

generally, and they have been followed, where relevant, in the preparation of this reprint:

o           the enacting words have been omitted

o           provisions that have been repealed or revoked are omitted, but a

note indicates the repealing enactment or revoking provision

o           footnotes are included to indicate where---

o  	     words, phrases, or provisions have been inserted or

substituted, the commencement date of the amend- ment, and the                                    	     particular enactment responsible for the change

o  references in a reprinted enactment to any repealed 	          	       enactment have been replaced with a reference to any  	                                                    	       enactment that replaces, or corresponds to, the repealed

enactment, in reliance in section 22(2) of the   	       	       interpretation Act 1999

o           any term used in a reprinted enactment that has been deemed, by

another enactment, to be read as if it were another term has

been replaced by that other term

o           references in a reprinted enactment to amounts in pounds,

shillings, and pence have been replaced by a reference to the

equivalent amount in decimal currency (dollars and cents), in

reliance on section 7 of the Decimal Currency Act 1964

o           where the principal enactment has made a textual amendment to

another enactment, and the amendment has been incorporated in a

reprint of that other enactment, the text of the amendment is   	     not reprinted in full in the reprint

o           where a provision of an amending enactment has made a textual

amendment to the principal enactment, the provision of the

amending  enactment is not reprinted.

4           Changes made under section 17C of the Acts and Regulations

Publication Act 1989

Section 17C of the Acts and Regulations Publication Act 1989 authorises the

making of editorial changes in a reprint so that, to the extent permitted, the format and style of the reprinted enactment is consistent with current

legislative drafting practice. Changes that would change the effect of the

legislation are not permitted.

A new format of legislation was introduced on 1 January 2000.

Changes to legislative drafting style have also been made since 1997, and are ongoing. To the extent permitted by section 17C of the Acts and Regulations Publication Act 1989, all legislation reprinted after 1 January 2000 is in the new format for legislation and reflects current legislative drafting practice.

In outline, the editorial changes made in reprints under the authority of

section 17C of the Acts and Regulations Publication Act 1989 are as follows, and they have been followed, where relevant, in the preparation of this reprint:

o           unnecessary referential words (such as "of this section" and

"of this Act") have been omitted

o           typeface and type size have been changed to accord with current

drafting practice (Times Roman, generally in 11.5 point with

13.5 point leading)

o           the setting out of provisions has been changed to accord with

current drafting practice. These changes include---

o  changes to the indenting and layout of provisions

o  the repositioning of section headings, so that the number and

heading appear above the section

o  the reformatting of definitions, so that the defined term

appears in bold type, without quotation marks

o           dates are expressed in a manner consistent with current

drafting practice (eg, a date formerly expressed as "the 1st

day of January 1999" is now expressed as "1 January 1999")

o           the date of assent has been repositioned so that it appears on

the front page of each Act

o           punctuation is consistent with current drafting practice (eg,

colons are not used after definitions)

o           Parts numbered with roman numerals have been replaced with

arabic numerals, and all cross-references changed accordingly

o           the case and appearance of letters and words have been changed

to accord with current drafting practice

o           headings have been reformatted (eg, headings where each word

formerly appeared with an initial capital letter followed by

small capitals have been changed so that the heading appears

in bold, with only the first word (and any proper names)

appearing with an initial capital)

o           letters appearing in small capitals in section and subsection

references have been changed to capital letters (eg, "section

7---a" is now expressed as "section 7A")

o           schedules have been renumbered (eg, Schedule 1 replaces First

Schedule), and all cross-references changed accordingly

o           running heads (the information that appears at the of each

page) have been altered to accord with current drafting practice.

Two-column schedules of consequential amendments, and schedules of repeals,

have been reformatted, and rearranged into alphabetical order (rather than

chronological), to accord with current drafting practice.

5           List of amendments incorporated in this reprint

(most recent first)

The Hazardous Substances (Minimum Degrees of Hazard) Regulations 2001 are

administered in the Ministry for the Environment.