Reprint as at 9 June 2005

Hazardous Substances (Identification) Regulations 2001

(SR 2001/124)

Silvia Cartwright, Governor-General

Order in Council

At Wellington this 28th day of May 2001

Present:

Her Excellency the Governor-General in Council

Pursuant to section 76(1)(b) of the Hazardous Substances and New Organisms Act

1996, Her Excellency the Governor-General, acting on the advice and with the

consent of the Executive Council (given on the recommendation of the Minister

for the Environment made in compliance with section 141(1) of that Act), makes

the following regulations.

Contents

1    Title

2    Commencement

3    Interpretation

4    Exclusion for substances used for motive power or control of vehicle,

aircraft, or ship

5    Exclusion for fuel gases

Part 1

Identification of hazardous substances

Duties

6    Identification duties of suppliers

7    Identification duties of persons in charge

Priority identifiers

8    Priority identifiers for corrosive substances

9    Priority identifiers for certain ecotoxic substances

10   Priority identifiers for explosive substances

11   Priority identifiers for flammable substances

12   Priority identifiers for organic peroxides

13   Priority identifiers for oxidising substances

14   Priority identifiers for certain toxic substances

15   Labelling under UN Model Regulations

16   Use of EEC pictogram for Dangerous for the Environment

17   Single indication can convey several matters

Secondary identifiers

18   Secondary identifiers for all hazardous substances

19   Secondary identifiers for corrosive substances

20   Secondary identifiers for ecotoxic substances

21   Secondary identifiers for explosive substances

22   Secondary identifiers for flammable substances

23   Secondary identifiers for organic peroxides

24   Secondary identifiers for oxidising substances

25   Secondary identifiers for toxic substances

26   Use of generic names

27   Concentrations

28   Indicating general degree of hazard by reference to packing group

Alternative or additional information

29   Alternative information in certain cases

30   Substances in multiple packaging

31   Alternative information when hazardous substances imported

Presentation requirements for identification information

32   Accessibility of priority identifiers

33   Accessibility of secondary identifiers

34   Comprehensibility of information

35   Clarity of information

36   Durability of information

Part 2

Documentation for hazardous substances in places of work

37   Documentation duties of suppliers

38   Documentation duties of persons in charge of places of work

39   General content requirements for documentation

40   Specific documentation requirements for corrosive substances

41   Specific documentation requirements for certain ecotoxic substances

42   Specific documentation requirements for explosive substances

43   Specific documentation requirements for flammable substances

44   Specific documentation requirements for organic peroxides

45   Specific documentation requirements for oxidising substances

46   Specific documentation requirements for toxic substances

47   Information not included in approval

48   Location and presentation requirements for documentation

49   Documentation requirements for vehicles

50   Documentation to be supplied on request

Part 3

Signage and advertising

Signage

51   Duties of persons in charge of places in respect of signage

52   Signage requirements

Advertising toxic substances

53   Advertising corrosive and toxic substances

Schedule 1

Distances for previewing and hearing information, documentation, and signage

Schedule 2

Quantities requiring documentation

Schedule 3

Quantities requiring signage

Regulations

1 Title

These regulations are the Hazardous Substances (Identification) Regulations

2001.

2 Commencement

These regulations come into force on 2 July 2001.

3 Interpretation

(1)  In these regulations, unless the context otherwise requires,---

Act    means the Hazardous Substances and New Organisms Act 1996

aerosol    has the meaning given to that term by clause 1 of Schedule 2 of the Hazardous Substances (Classification) Regulations 2001

average eyesight    means the ability to identify random letters at level 6 on the N point scale; and N point scale means the optometry scale described in Grazia, R P, 1996, Vision and Reading, published by Mosby

category   means a category under the classification system described in

regulation 4 of the Hazardous Substances (Classification) Regulations 2001

class   means a class under the classification system described in regulation 4 of the Hazardous Substances (Classification) Regulations 2001

damage    includes destruction

directive 92/32/EEC    means Council of the European Communities Directive of 30 April 1992 amending for the Seventh time Directive 67/548/EEC on the

Approximation of the Laws, Regulations and Administrative Provisions relating to the classification, packing and labelling of Dangerous Substances

documentation    includes information recorded electronically

general degree, in relation to a hazard, means a general indication of its

category (for example, "very high"), whether given in words or by any other

means

general type, in relation to a hazardous substance, means a general indication of its subclass (for example, "dangerous when wet"), whether given in words or by any other means

harm, in relation to human beings or other living organisms, means death,

injury, or ill health

hazard classification   means a classification under the classification system described in regulation 4 of the Hazardous Substances (Classification) Regulations 2001

information    includes an audio message, bar code, colour coding, instruction,label, microchip, name, phrase, picto- gram, sign, or symbol

organic peroxide    means an organic peroxide that meets the threshold criteria of the Hazardous Substances (Minimum Degrees of Hazard) Regulations 2001 for substances with oxidising properties

oxidising substance    means a substance that---

(a)  meets the threshold criteria of the Hazardous Substances (Minimum Degrees of Hazard) Regulations 2001 for substances with oxidising properties; but

(b)  is not an organic peroxide

packaging    and outer packaging have the meanings given to those terms by

regulation 3 of the Hazardous Substances (Packaging) Regulations 2001

required documentation    means the documentation required by Part 2

required identification information    means the identification information

required by Part 1

subclass   means a subclass under the classification system described in

regulation 4 of the Hazardous Substances (Classification) Regulations 2001

(2)  In these regulations, unless the context otherwise requires,---

(a)  aggregate water capacity, place, and vehicle have the meanings given to those terms by regulation 3 of the Hazardous Substances (Classes 1 to 5 Controls) Regulations 2001:

(b)  gas, liquid, solid, and UN Model Regulations have the meaning given to those terms by regulation 3 of the Hazardous Substances (Minimum Degrees of Hazard) Regulations 2001:

(c)  corrosive substance, ecotoxic substance, explosive substance,

flammable substance, and toxic substance have the meanings given to those terms by regulation 3 of the Hazardous Substances (Classification) Regulations 2001.

Regulation 3(1) category: substituted, on 26 March 2004, by regulation 3(1) of the Hazardous Substances (Identification) Amendment Regulations 2004

(SR 2004/45).

Regulation 3(1) class: inserted, on 26 March 2004, by regulation 3(2) of the Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

Regulation 3(1) hazard classification: substituted, on 26 March 2004, by

regulation 3(3) of the Hazardous Substances (Identification) Amendment

Regulations 2004 (SR 2004/45).

Regulation 3(1) subclass: substituted, on 26 March 2004, by regulation 3(4) of the Hazardous Substances (Identification) Amendment Regulations 2004

(SR 2004/45).

4 Exclusion for substances used for motive power or control of vehicle,

aircraft, or ship

(1)  The requirements of these regulations do not apply to any substance that is required for the motive power or control of a motor vehicle, aircraft, or ship, and is contained in the fuel system, electrical system, or control system of the vehicle, aircraft, or ship.

(2)  This regulation overrides all other provisions of these regulations.

Regulation 4(1): word inserted, on 26 March 2004, by regulation 4 of the

Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

5 Exclusion for fuel gases

(1)  The requirements of these regulations do not apply to any fuel gas

supplied or used in a gas distribution system, gas appliance, or gas

installation, when subject to the Gas Act 1992.

(2)  In subclause (1),---

fuel gas    means any fuel supplied through pipes or

in containers that is a gas at 15oC and 101.3 kPa absolute pressure; and

includes---

(a)  biogas, coal gas, liquified petroleum gas, natural gas, oil gas,

producer gas, refinery gas, and reformed natural gas; and

(b)  any gaseous substance that the Governor-General declares by Order in Council under the Gas Act 1992 to be a gas for the purposes of that Act; and

(c)  any gaseous substance or mixture whose composition complies with

regulations under the Gas Act 1992 allowing it to be used as fuel; and

(d)  any mixture of fuel gases

gas appliance    has the meaning given to that term by section 2(1) of the Gas Act 1992

gas distribution system    means a distribution system within the meaning of section 2(1) of the Gas Act 1992

gas installation    has the meaning given to that term by section 2(1) of the Gas Act 1992.

(3)  This regulation overrides all other provisions of these regulations.

Part 1

Identification of hazardous substances

Duties

6 Identification duties of suppliers

A person must not supply a hazardous substance to any other person unless---

(a)  either---

(i)     it is identified as required by this Part; or

(ii)    on sale or supply, it is placed in or delivered into a place or container that is identified as required by this Part; and

(b)  there is not on it or its packaging, or with it, any information

suggesting that it belongs to a class or subclass it does not in fact belong to.

Regulation 6(b): words inserted, on 26 March 2004, by regulation 5 of the

Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

7 Identification duties of persons in charge

A person who is in charge of a hazardous substance must ensure that---

(a)  it complies with regulations 8 to 30, and regulations 32 and 33; and

(b)  there is not on it or its packaging, or with it, any information

that suggests that it belongs to a class or subclass it does not in fact belong to.

Regulation 7(b): words inserted, on 26 March 2004, by regulation 6 of the

Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

Priority identifiers

8 Priority identifiers for corrosive substances

A corrosive substance must be identified by the following information:

(a)  an indication that it is corrosive; and

(b)  unless it is in a place of work, or part of a place of work, where children may not lawfully be present, an indication that access by children must be prevented.

9 Priority identifiers for certain ecotoxic substances

An ecotoxic substance (other than a substance with a hazard classification

9.1D, 9.2D, or 9.3C) must be identified by an indication that it is ecotoxic.

10 Priority identifiers for explosive substances

An explosive substance must be identified by the following information:

(a)  an indication that it is explosive; and

(b)  an indication of its hazard classification.

11 Priority identifiers for flammable substances

A flammable substance must be identified by the following information:

(a)  an indication that it is flammable:

(b)  an indication of whether it is a gas, an aerosol, a liquid, or a

solid:

(c)  if it is a liquid, an indication of its general degree of hazard

(for example, "highly flammable"):

(d)  if it is a solid, an indication of its general type of hazard (for example, "dangerous when wet").

12 Priority identifiers for organic peroxides

An organic peroxide must be identified by the following information:

(a)  an indication that it has oxidising properties:

(b)  an indication that it is an organic peroxide.

13 Priority identifiers for oxidising substances

An oxidising substance must be identified by the following information:

(a)  an indication that it has oxidising properties:

(b)  if it is a gas, an indication that it is a gas.

14 Priority identifiers for certain toxic substances

A toxic substance with a hazard classification of 6.1 (other than a toxic

substance with a hazard classification of 6.1E that is not intended to be sold to members of the public) must be identified by the following information:

(a)  an indication that it is toxic:

(b)  unless it is a toxic substance with a hazard classification of 6.1D in a place of work, or part of a place of work, where members of the public may not lawfully be present, an indication of its general degree and type of hazard (for example, "highly poisonous"):

(c)  information indicating the need to restrict access by children.

15 Labelling under UN Model Regulations

In the case of a corrosive substance, an explosive substance, a flammable

substance, or an oxidising substance or organic peroxide, the assignment to it, in accordance with the UN Model Regulations, of a label (as defined in

paragraph 5.2.2 of those model regulations) is an acceptable means of complying with whichever of regulations 8 to 14 applies to it.

16 Use of EEC pictogram for Dangerous for the Environment

In the case of an ecotoxic substance, the assignment to it of the pictogram for "Dangerous for the Environment", as described in Directive 92/32/EEC, is an acceptable means of complying with regulation 9.

17 Single indication can convey several matters

In any case, a single indication is capable of conveying 2 or more matters

required by any of regulations 8 to 14.

Secondary identifiers

18 Secondary identifiers for all hazardous substances

In addition to any information required by regulations 8 to 14, a hazardous

substance must be identified by the following information:

(a)  an indication (which may include its common name, chemical name, or registered trade name) that unequivocally identifies it:

(b)  enough information to enable its New Zealand importer, supplier, or manufacturer to be contacted, either in person or by telephone:

(c)  in the case of a substance that, when in a closed container, is

likely over time to become more hazardous, or develop additional hazardous

properties, or become a hazardous substance of a different class or subclass,---

(i)     a description of each likely change; and

(ii)    the date by which it is likely to occur.

Regulation 18(c): words inserted, on 26 March 2004, by regulation 7 of the

Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

19 Secondary identifiers for corrosive substances

In addition to any information required by regulations 8 and 18, a corrosive substance must be identified by the following information:

(a)  an indication of its general degree and general type of

corrosiveness (for example, "highly corrosive to skin"):

(b)  an indication of the circumstances in which it may harm skin or eye tissue:

(c)  an indication of the circumstances in which it may damage metallic objects:

(d)  an indication of the kind and extent of the harm it is likely to

cause to skin or eye tissue:

(e)  an indication of the kind and extent of the damage it is likely to cause to metallic objects:

(f)  if the substance contains any ingredient in such a concentration

that the ingredient would, independently of any other ingredient, cause the

substance to be classified as class 8.2 or class 8.3, in respect of each such ingredient,---

(i)     its common or chemical name; and

(ii)    a statement of its concentration in the substance.

20 Secondary identifiers for ecotoxic substances

In addition to any information required by regulations 9 and 18, an ecotoxic substance must be identified by the following information:

(a)  an indication of the circumstances in which it may harm living

organisms:

(b)  an indication of the kind and extent of the harm it is likely to

cause to living organisms:

(c)  an indication of the steps to be taken to prevent harm to living

organisms:

(d)  in the case only of an ecotoxic substance that has a hazard

classification of 9.1A, 9.1B, or 9.1C, an indication of its general degree of hazard and general type of hazard (for example, "very ecotoxic to aquatic life"):

(e)  in the case only of an ecotoxic substance that has a hazard

classification of 9.2A, 9.2B, 9.2C, 9.3A, 9.3B, 9.4A, 9.4B, or 9.4C, an

indication of its general type of hazard (for example, "ecotoxic in the soil environment").

21 Secondary identifiers for explosive substances

In addition to any information required by regulations 10 and 18, an explosive substance must be identified by the following information:

(a)  an indication of the circumstances in which it may be caused to

explode unintentionally:

(b)  an indication of the likely effect of an unintentional explosion:

(c)  an indication of the steps to be taken to prevent an unintentional explosion:

(d)  if an explosive is approved by the Authority for use in underground mining operations where flammable atmospheres are present, an identifier approved for the purpose by the Authority.

Regulation 21(d): added, on 28 August 2003, by regulation 3 of the Hazardous Substances (Identification) Amendment Regulations 2003 (SR 2003/180).

22 Secondary identifiers for flammable substances

In addition to any information required by regulations 11 and 18, a flammable substance must be identified by the following information:

(a)  an indication of its general degree and general type of hazard (for example, "highly flammable liquid"):

(b)  an indication of the circumstances in which it may be ignited

unintentionally:

(c)  an indication of the likely effect of an unintentional ignition:

(d)  an indication of the steps to be taken to prevent an unintentional ignition.

23 Secondary identifiers for organic peroxides

In addition to any information required by regulations 12 and 18, an organic peroxide must be identified by the following information:

(a)  an indication of the general degree of hazard it has that

contributes to its having a hazard classification in Class 5:

(b)  an indication of the circumstances in which it may, unintentionally,burn, accelerate a fire, or thermally decompose:

(c)  an indication of the likely effect of an unintentional combustion, acceleration of a fire, or thermal de- composition:

(d)  an indication of the steps to be taken to prevent an unintentional combustion, acceleration of a fire, or thermal decomposition:

(e)  the control temperature specified in regulation 110 of the Hazardous Substances (Classes 1 to 5 Controls) Regulations 2001.

Regulation 23(e): expression substituted, on 26 March 2004, by regulation 8 of the Hazardous Substances (Identification) Amendment Regulations 2004

(SR 2004/45).

24 Secondary identifiers for oxidising substances

In addition to any information required by regulations 13 and 18, an oxidising substance must be identified by the following information:

(a)  an indication of the general degree of hazard it has that

contributes to its having a hazard classification in Class 5 (for example,

"highly hazardous oxidising liquid"):

(b)  an indication of the circumstances in which it may, unintentionally, burn, accelerate a fire, or thermally decompose:

(c)  an indication of the likely effect of an unintentional combustion, acceleration of a fire, or thermal de- composition:

(d)  an indication of the steps to be taken to prevent an unintentional combustion, acceleration of a fire, or thermal decomposition.

25 Secondary identifiers for toxic substances

In addition to any information required by regulations 14 and 18, a toxic

substance must be identified by the following information:

(a)  an indication of its general degree and general type of hazard (for example, "mild skin irritant"):

(b)  an indication of the circumstances in which it may harm human

beings:

(c)  an indication of the kinds of harm it may cause to human beings and, in respect of each kind (other than death), its likely extent:

(d)  an indication of the steps to be taken to prevent harm to human

beings:

(e)  information identifying, by its common or chemical name, every

ingredient that would, independently of any other ingredient, give the

substance a hazard classification of 6.1A, 6.1B, 6.1C, 6.5, 6.6, 6.7, 6.8, or 6.9, and the concentration of that ingredient in the substance:

(f)  information identifying every ingredient (other than an ingredient referred to in paragraph (e)) that would, independently of any other ingredient, give the substance a hazard classification of 6.1D, and the concentration of the ingredient that would contribute the most to that

classification.

26 Use of generic names

(1)  A generic name may be used to identify a group of ingredients required by regulation 19(f) to be identified, if---

(a)  they are part of a complex mixture and have not been individually quantified; and

(b)  the name applies to the group.

(2)  A generic name may be used to identify a group of ingredients each of which is otherwise required by regulation 25(e) to be identified by its common or chemical name, if---

(a)  they are part of a complex mixture and have not been individually quantified; and

(b)  the name applies to the group.

(3)  A generic name may be used to identify collectively several ingredients required by regulation 25(f) to be identified, if the name applies to each of them.

(4)  A generic name may be used to identify an ingredient required by

regulation 25(f) to be identified, if section 55 or section 57 of the Act

applies to it.

(5)  The generic name must identify the key chemical entities and functional groups in the ingredients that contribute to their hazardous properties.

27 Concentrations

For the purposes of regulations 19 and 25, the concentration of an ingredient in a substance may be stated as a range if---

(a)  the range does not have the effect that the substance falls into

more than 1 category of a subclass; and

(b)  either---

(i)     section 55 or section 57 of the Act applies to the

ingredient; or

(ii)    the Authority approved the concentration in that range.

28 Indicating general degree of hazard by reference to packing group

The identification, in accordance with the UN Model Regulations, of the

packing group to which the substance concerned is assigned is an acceptable

means of complying with a requirement under regulation 19(a), regulation 22(a),

regulation 23(a), regulation 24(a), or regulation 25(a) to indicate a

substance's general degree of hazard.

Alternative or additional information

29 Alternative information in certain cases

(1)  In the case of a hazardous substance enclosed in a container that is

fixed in one place, it is a sufficient compliance with regulations 8 to 25 if there is at all times available to people near the container information

(accessible as required by regulation 33 and meeting the requirements for

comprehensibility and clarity of regulation 51, with a specified minimum

distance of 10 m)---

(a)  identifying the general type and general degree of hazard of the

substance in accordance with regulations 8 to 14 and regulations 19 to 25; and

(b)  if it is an explosive substance, a flammable substance, or an

oxidising substance or organic peroxide, describing, as the case requires,---

(i)     the steps to be taken to prevent an unintentional explosion:

(ii)    the steps to be taken to prevent an unintentional ignition:

(iii)   the steps to be taken to prevent an unintentional combustion, acceleration of a fire, or thermal decomposition.

(2)  In the case of a hazardous substance enclosed in a bulk transport

container, it is a sufficient compliance with regulations 8 to 25 if there is at all times available to any people near the container information that

complies with the requirements of---

(a)  Land Transport Rule 45001: Dangerous Goods 1999; or

(b)  Civil Aviation Act 1990 (Rule, Part 92); or

(c)  Maritime Transport Act 1994 (Maritime Rule 24A).

Regulation 29(1)(a): words substituted, on 26 March 2004, by regulation 9 of the Hazardous Substances (Identification) Amendment Regulations 2004

(SR 2004/45).

30 Substances in multiple packaging

(1)  This subclause applies to a hazardous substance if---

(a)  it is identified as required by this Part; but

(b)  some or all of the required information is obscured by outer

packaging.

(2)  The outer packaging of a hazardous substance to which subclause (1)

applies must bear---

(a)  the information required by whichever of regulations 8 to 14 applies to the substance; or

(b)  the labelling or marking required by---

(i)     Land Transport Rule 45001: Dangerous Goods 1999; or

(ii)    Civil Aviation Act 1990 (Rule, Part 92); or

(iii)   Maritime Transport Act 1994 (Maritime Rule 24A); or

(c)  in the case of an ecotoxic substance, the pictogram for "Dangerous for the Environment" as described in directive 92/32/EEC; or

(d)  the relevant class or subclass label assigned by the UN Model

Regulations.

Regulation 30(2)(d): words inserted, on 26 March 2004, by regulation 10 of the Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

31 Alternative information when hazardous substances imported

(1)  This subclause applies to a hazardous substance that---

(a)  has been imported into New Zealand in a closed package or in a

freight container; and

(b)  is being carried from the place of importation to the destination stated in its importation documentation without having been removed from that package or container.

(2)  Before a hazardous substance to which subclause (1) applies arrives at the destination stated in its importation documentation (and for any reasonable period after it arrives that is necessary to arrange actual compliance with regulations 8 to 25), it is a sufficient compliance with those regulations if the package or container concerned complies with the requirements of Land Transport Rule 45001: Dangerous Goods 1999.

Presentation requirements for identification information

32 Accessibility of priority identifiers

Until the substance concerned has been treated or disposed of in accordance

with the Hazardous Substances (Disposal) Regulations 2001, the information

required by regulations 8 to 14 must be so provided that it is available to a person handling the substance within 2 seconds.

33 Accessibility of secondary identifiers

Until the substance concerned has been treated or disposed of in accordance

with the Hazardous Substances (Disposal) Regulations 2001, the information

required by regulations 18 to 25 must be so provided that it is available to a person handling the substance within 10 seconds.

34 Comprehensibility of information

(1)  The following provisions apply to all required identification

information that is provided in words (whether written or oral)---

(a)  it must be readily understandable as required by subclause (4):

(b)  it must be in the English language:

(c)  no abbreviation or acronym may be used unless---

(i)     it is in common English usage; and

(ii)    the term of which it is an abbreviation or acronym has

already been used at least once in full.

(2)  All required identification information that is provided in pictorial form must, when considered with the colours in which it is presented and any words presented with it, be readily understandable.

(3)  Required identification information that is affixed to or forms part of a package or container must comply with subclauses (1) and (2) both when the package or container is open and when it is closed.

(4)  For the purposes of subclause (1)(a), identification information is

readily understandable if, and only if,---

(a)  a sample of 50 or more people has been randomly chosen from members of the general public who---

(i)     have a reading age of at least 12 years; and

(ii)    have no particular education or training in relation to

hazardous substances; and

(b)  the sample has been provided with the information for a period of 5 minutes; and

(c)  the members of the sample have then been asked more than 5 and fewer than 11 questions about the information in an interview or test lasting no more than 10 minutes; and

(d)  at least 80% of the responses were an accurate interpretation of the information.

35 Clarity of information

(1)  All required identification information provided in written or pictorial form must be able to be easily read or perceived by a person with average eyesight (whether average eyesight without corrective lenses or eyesight corrected to average levels with the help of corrective lenses) under the conditions stated in subclause (3), whether or not the person has any form of inability to distinguish or correctly identify colours.

(2)  All required identification information provided in audible form must be able to be easily heard by a person with average hearing (whether average hearing without a corrective hearing device or hearing corrected to average levels with the help of corrective hearing device) under the conditions stated in subclause (3).

(3)  The conditions referred to in subclauses (1) and (2) are---

(a)  a minimum lighting level of 150 lx, in the absence of glare,

reflection, flicker, or stroboscopic effects; and

(b)  a background noise level of 50 dB and a minimum signal-to-noise

ratio of -3 dB; and

(c)  at a distance of not less than the values specified in Schedule 1.

(4)  Required identification information that is affixed to or forms part of a package or container must comply with subclause (1) both when the package or container is open and when it is closed.

(5)  For the purposes of subclause (3), when printed on a background of a

suitable contrasting colour,---

(a)  sans serif lettering whose lower case letters are at least 1 mm high is considered to be readable over distances of up to 0.2 m; and

(b)  sans serif lettering whose lower case letters are at least 1.5 mm high is considered to be readable over distances of up to 0.3 m.

(6)  In subclause (5), suitable contrasting colour, in relation to any

lettering, means a colour that---

(a)  contrasts with the colour of the lettering; and

(b)  does not effect for a person with any form of inability to

distinguish or correctly identify colours a contrast less than that effected for a person without such an inability.

36 Durability of information

(1)  All required identification information must continue to meet the

requirements of regulation 35 throughout the lifetime of the substance

concerned while subjected to---

(a)  a normal range of temperatures and humidity levels; and

(b)  the wear and tear of normal storage, handling, and use.

(2)  If required identification information is exposed directly to the

weather, or is likely to be exposed directly to the weather throughout the

lifetime of the substance concerned, it must be capable of continuing to meet the requirements of regulation 35---

(a)  while continuously exposed to radiation equivalent to the usual

solar radiation in an unshaded position in New Zealand for a 90-day period

between November and March; and

(b)  while continuously exposed to temperatures in the range of 0oC to 50oC and humidity in the range of 10% to 95% for a 90-day period when,---

(i)     for at least of 10% of the period, the temperature is 0oC;

and

(ii)    for at least of 10% of the period, the temperature is 50oC;

and

(iii)   for at least of 10% of the period, the humidity is 10%; and

(iv)    for at least of 10% of the period, the humidity is 95%; and

(c)  while continuously in contact with water for 30 days; and

(d)  while continuously in contact with the hazardous substance concerned for 4 minutes at 25oC.

(3)  If required identification information is not exposed directly to the weather and not likely to be exposed directly to the weather throughout the lifetime of the substance concerned, it must be capable of continuing to meet the requirements of regulation 35---

(a)  while continuously exposed to radiation equivalent to the usual

solar radiation in an unshaded position in any place in New Zealand for any

30-day period between November and March; and

(b)  while continuously exposed to temperatures in the range of 0oC to 50oC and humidity in the range of 10% to 95% for a 30-day period when,---

(i)     for at least of 10% of the period, the temperature is 0oC;

and

(ii)    for at least of 10% of the period, the temperature is 50oC;

and

(iii)   for at least of 10% of the period, the humidity is 10%; and

(iv)    for at least of 10% of the period, the humidity is 95%; and

(c)  while continuously in contact with the hazardous substance concerned for 4 minutes at 25oC.

(4)  Information provided as a label that passes the tests specified in

Appendix C --- Method for Determining Indelibility of Print of Australian

Standard 2581 --- 1998 Pressure sensitive adhesive labels for general purpose use meets the requirements of paragraphs (a), (b), and (c), of subclause (2).

(5)  If required identification information is affixed to or forms part of a package or container, the durability requirements of subclauses (2)(b) and (3)(b) do not have to be met under temperatures or other environmental

conditions prohibited by the Hazardous Substances (Classes 1 to 5 Controls)

Regulations 2001.

(6)  In the case of a hazardous substance required by its manufacturer to be stored at temperatures that are within a range that is or extends outside 0oC to 50oC, the identification information must (instead of satisfying the requirements of subclause (3)(b)) be capable of continuing to meet the requirements of regulation 24 while continuously exposed to temperatures within that range for a 30-day period.

(7)  Required identification information does not fail to comply with

subclauses (2)(d) or subclause (3)(c) just because the substance concerned

forms an opaque layer over it.

(8)  Any packaging in direct contact with a class 6.1 substance must be

permanently identified as having contained a toxic substance unless---

(a)  the substance as packaged is restricted to places of work; or

(b)  the substance is a toxic substance of class 6.1E.

Part 2

Documentation for hazardous substances in places of work

37 Documentation duties of suppliers

A person must comply with regulations 39 to 46 when selling or supplying to

another person a quantity of a hazardous substance of a particular hazard

classification equal to or greater than that specified for hazardous substances of that classification in Schedule 2, if---

(a)  the person is satisfied on reasonable grounds that the other person is likely to use the substance in a place of work; and

(b)  the person has never previously---

(i)     sold or supplied a quantity of the substance in that state to the other person in circumstances requiring compliance with regulations 39 to 48; and

(ii)    complied with those regulations in relation to selling or

supplying it.

Regulation 37(b)(i): words substituted, on 26 March 2004, by regulation 11 of the Hazardous Substances (Identification) Amendment Regulations 2004

(SR 2004/45).

38 Documentation duties of persons in charge of places of work

(1)  This regulation applies to a place of work---

(a)  if there is held in it any quantity of a hazardous substance of a particular hazard classification in respect of which Schedule 2 specifies "any quantity"; or

(b)  if there is held in it (whether in a single container or 2 or more containers) an aggregate quantity of a hazardous substances of hazard

classification 3.1D, 5.2G, 6.1E, 6.3A, 6.3B, 6.4A, 9.1D, or 9.2D, equal to or greater than the quantity specified for hazardous substances of that

classification in Schedule 2; or

(c)  if there is held in it in a single container a quantity of a

hazardous substance of a particular hazard classification (other than a

hazardous substance of hazard classification 3.1D, 5.2G, 6.1E, 6.3A, 6.3B,

6.4A, 9.1D, or 9.2D) equal to or greater than the quantity specified for

hazardous substances of that classification in Schedule 2; or

(d)  if---

(i)     there is held in it in 2 or more containers an aggregate

quantity of hazardous substances of a particular hazard classification (other than hazardous substances of hazard classification 3.1D, 5.2G, 6.1E, 6.3A, 6.3B, 6.4A, 9.1D, or 9.2D) equal to or greater than 10 times the quantity specified for hazardous substances of that classification in Schedule 2; and

(ii)    none of the individual containers contains a quantity of a hazardous substance of that hazard classification equal to or greater than the quantity specified for hazardous substances of that classification in Schedule 2.

(2)  If this regulation applies to a place of work, a person in charge of the place must ensure that---

(a)  every person handling the substance has access to the documentation required for each hazardous substance concerned; and

(b)  there is not in that documentation any information that suggests

that the substance belongs to a class or subclass it does not in fact belong to.

Regulation 38(1)(b): expression inserted, on 26 March 2004, by regulation 12(1) of the Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

Regulation 38(1)(c): expression inserted, on 26 March 2004, by regulation 12(1) of the Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

Regulation 38(1)(d)(i): expression inserted, on 26 March 2004, by

regulation 12(1) of the Hazardous Substances (Identification) Amendment

Regulations 2004 (SR 2004/45).

Regulation 38(2)(b): words substituted, on 26 March 2004, by regulation 12(2) of the Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

39 General content requirements for documentation

(1)  The required documentation---

(a)  must unequivocally identify the substance concerned; and

(b)  must include---

(i)     the substance's chemical name; and

(ii)    the identification as required for the substance's approval;

and

(c)  to the extent that the substance has, or has had assigned to it, any of the following information, must include that information:

(i)     the substance's registered trade name or names:

(ii)    all generally known common names:

(iii)   the substance's UN number:

(iv)    any Chemical Abstract Services number allocated to the

substance.

(2)  The required documentation must---

(a)  describe the physical state, colour, and odour of the substance; and

(b)  if the substance's physical state may alter over the expected range of place of work temperatures, must describe---

(i)     the temperatures at which the changes in physical state may occur; and

(ii)    the changes in physical state that may occur at each of those temperatures; and

(c)  in the case of a substance that, when in a closed container, is

likely over time to become more hazardous, or develop additional hazardous

properties, or become a hazardous substance of a different class,---

(i)     a description of each likely change; and

(ii)    the date by which it is likely to occur.

(3)  The required documentation must contain enough information to enable the New Zealand importer, supplier, or manufacturer of the substance to be

contacted, either in person or by telephone.

(4)  The required documentation must---

(a)  state the date on which it was prepared; and

(b)  include all emergency management and disposal information required by the Hazardous Substances (Emergency Management) Regulations 2001 or the Hazardous Substances (Disposal) Regulations 2001.

(5)  The requirements of regulation 19(f) or (as the case requires)

regulation 25(e) apply to all documentation; but any ingredient required by

that provision to be identified (other than an ingredient to which regulation 26 applies) must also be identified by any Chemical Abstract Services number allocated to it.Regulation 39(5): expression substituted, on 26 March 2004, by regulation 13 of the Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

40 Specific documentation requirements for corrosive substances

The required documentation for a corrosive substance must include the following hazardous property and precautionary information:

(a)  its general degree and general type of hazard:

(b)  a full description of the circumstances in which it may cause

unintentional damage to tissue or metallic objects:

(c)  a full description of the potential consequences of any damage it may cause to tissue or metallic objects:

(d)  a full description of the steps to be taken to prevent unintentional damage to tissue or metallic objects:

(e)  if it is in subclass 8.2 or 8.3, its pH or pH range.

41 Specific documentation requirements for certain ecotoxic substances

The required documentation for an ecotoxic substance (other than an ecotoxic substance with a hazard classification of 9.1D, 9.2D, or 9.3C) must include the following hazardous property and precautionary information:

(a)  its general degree and general type of hazard:

(b)  a full description of the circumstances in which and extent to which it may harm living organisms:

(c)  a full description of the steps to be taken to prevent harm to

living organisms:

(d)  a summary of the available acute and chronic data used to define the subclass or subclasses in which it is classified:

(e)  its bio-concentration factor or octanol-water partition coefficient:

(f)  its expected soil or water degradation rate, expressed as either---

(i)     the number of days required to achieve a 50% or 90% reduction of the original substance concentration; or

(ii)    the results of a 28-day ready biodegradability test:

(g)  any environmental effect level (EEL) specified in the Authority

approval of the substance or, if it is a mixture, for any of its components.

42 Specific documentation requirements for explosive substances

The required documentation for an explosive substance must include the

following hazardous property and precautionary information:

(a)  its general degree and general type of hazard:

(b)  a full description of the circumstances in which it may be caused to explode unintentionally:

(c)  the likely effect of an unintentional explosion:

(d)  a full description of the steps to be taken to prevent an

unintentional explosion.

43 Specific documentation requirements for flammable substances

The required documentation for a flammable substance must include the following hazardous property and precautionary information:

(a)  its general degree and general type of hazard:

(b)  a full description of the circumstances in which it may be ignited unintentionally:

(c)  the likely effect of an unintentional ignition:

(d)  a full description of the steps to be taken to prevent an

unintentional ignition:

(e)  if it is a gas, its lower and upper explosive limits, expressed as volume percentages in air:

(f)  if it is a liquid,---

(i)     its lower and upper explosive limits, expressed as volume

percentages in air; or

(ii)    its flash point, the methodology used to determine its flash point, and its auto-ignition temperature:

(g)  if it is a self reactive solid, its self-acceleration decomposition temperature and heat of decomposition per unit mass.

44 Specific documentation requirements for organic peroxides

The required documentation for an organic peroxide must include the following hazardous property and precautionary information:

(a)  its general degree and general type of hazard:

(b)  a full description of the circumstances in which it may,

unintentionally, burn, accelerate a fire, or thermally decompose:

(c)  the likely effect of an unintentional combustion, acceleration of a fire, or thermal decomposition:

(d)  a full description of the steps to be taken to prevent an

unintentional combustion, acceleration of a fire, or thermal decomposition:

(e)  its self-acceleration decomposition temperature:

(f)  its emergency temperature and control temperature as specified in the Hazardous Substances (Classes 1 to 5 Controls) Regulations 2001.

45 Specific documentation requirements for oxidising substances

The required documentation for an oxidising substance must include the

following hazardous property and precautionary information:

(a)  its general degree and general type of hazard:

(b)  a full description of the circumstances in which it may,

unintentionally, burn, accelerate a fire, or thermally decompose:

(c)  the likely effect of an unintentional combustion, acceleration of a fire, or thermal decomposition:

(d)  a full description of the steps to be taken to prevent an

unintentional combustion, acceleration of a fire, or thermal decomposition.

46 Specific documentation requirements for toxic substances

The required documentation for a toxic substance must include the following

hazardous property and precautionary information:

(a)  its general degree and general type of hazard:

(b)  a full description of the circumstances in which it may harm human beings:

(c)  the kinds of harm it may cause to human beings:

(d)  a full description of the steps to be taken to prevent its harming human beings:

(e)  if it is a gas or an aerosol, its vapour pressure, and the

temperature at which that pressure was measured:

(f)  if it will be a liquid during its use, the percentage of volatile substances in it, and the temperature at which that percentage was measured:

(g)  a summary of the available acute and chronic data used to define the subclass or subclasses in which it is classified:

(h)  the symptoms or signs of injury or ill health associated with each likely route of exposure:

(i)  the dose, concentration, or conditions of exposure, likely to

cause injury or ill health:

(j)  any tolerable exposure limit (TEL) or workplace exposure standard (WES) specified in the Authority approval of the substance or, if it is a mixture, for any of its components.

47 Information not included in approval

If information relating to any matter that is required by any of

regulations 39 to 46 was not included in the information used for the approval of a hazardous substance by the Authority, the requirements of that regulation in relation to that matter are met by---

(a)  stating the matter; and

(b)  indicating that no information relating to it is appli- cable.

48 Location and presentation requirements for documentation

(1)  The documentation required by this Part must be so provided that the

information it contains is available to a person handling the substance

concerned within 10 minutes.

(2)  The documentation must be readily understandable by any fully-trained worker required to have access to it.

(3)  Regulation 35 applies to the required documentation, but as if the

references to distances in subclauses (3) and (5) were references to a distance of not less than 0.3 m.

(4)  For the purposes of these regulations, documentation is readily

understandable if, and only if,---

(a)  a sample of 20 or more people has been randomly chosen from fully trained workers chosen from sectors where the use of the substance concerned is most likely to occur; and

(b)  the sample has been provided with the documentation for a period of 5 minutes; and

(c)  the members of the sample have then been asked more than 5 and fewer than 11 questions about the documentation in an interview or test lasting no more than 10 minutes; and

(d)  at least 80% of the responses were an accurate interpretation of the documentation.

49 Documentation requirements for vehicles

(1)  While a hazardous substance is being transported by land, it does not have to comply with regulations 39 to 46 or regulation 48 if there is in the vehicle concerned documentation complying with Land Transport Rule 45001: Dangerous Goods 1999 (made under the Land Transport Act 1998).

(2)  While a hazardous substance is being transported by sea, it does not

have to comply with regulations 39 to 46 or regulation 48 if there is in the ship concerned documentation complying with Maritime Rule 24A Carriage of Cargoes --- Dangerous Goods 1999 (made under Part 3 of the Maritime Transport Act 1994).

(3)  While a hazardous substance is being transported by air, it does not

have to comply with regulations 39 to 46 or regulation 48 if there is

in the aircraft concerned documentation complying with Civil Aviation Rule

92 --- Carriage of Dangerous Goods (made under Part 3 of the Civil Aviation Act 1990).

50 Documentation to be supplied on request

(1)  Subsection (2) applies to a person in charge of a place of work where a hazardous substance is stored or used.

(2)  A person who manufactures or supplies a hazardous substance in New

Zealand or imports a hazardous substance into New Zealand---

(a)  must, if asked to do so by any person to whom this subclause

applies, give that person the required documentation; and

(b)  must ensure that it meets the presentation requirements of

regulation 48.

Part 3

Signage and advertising

Signage

51 Duties of persons in charge of places in respect of signage

(1)  The person in charge of a place where the quantities of any hazardous substances of a hazard classification specified in Schedule 3 exceed the amount specified in that schedule for hazardous substances of that classification must ensure that---

(a)  the signage required by regulation 52 is provided; and

(b)  its content, presentation and positioning comply with that

regulation; and

(c)  it meets the requirements for comprehensibility set out in

subclauses (1) and (2) of regulation 34

(2)  Subclauses (1), (3), and (5) of regulation 35 apply to the required

signage, but as if the distances referred to in regulation 35(3) were a

distance of not less than 10 m.

(3)  For the purposes of this regulation, signage is readily understandable if, and only if,---

(a)  a sample of 50 or more people has been randomly chosen from members of the general public who---

(i)     have a reading age of at least 12 years; and

(ii)    have no particular education or training in relation to

hazardous substances; and

(b)  the sample has been provided with the signage for a period of 2

minutes; and

(c)  the members of the sample have then been asked more than 3 and fewer than 11 questions about the signage in an interview or test lasting no more than 5 minutes; and

(d)  at least 80% of the responses were an accurate interpretation of the signage.

(4)  Subsections (1) to (3) do not apply to a place that is---

(a)  a vehicle that is subject to the Land Transport Act 1998; or

(b)  a ship that is subject to the Maritime Transport Act 1994; or

(c)  an aircraft that is subject to the Civil Aviation Act 1990.

52 Signage requirements

(1)  If hazardous substances are located in a building (but not in a

particular room or compartment within it), there must be positioned at every vehicular and pedestrian access to the building, and every vehicular and pedestrian access to land where the building is located, signage---

(a)  stating that hazardous substances are present; and

(b)  stating the general type of hazard of each of them; and

(c)  Revoked.

(2)  If hazardous substances are located in a particular room or compartment within a building, there must be positioned at each entrance to the room or compartment signage complying with subsection (4).

(3)  If hazardous substances are located in an outdoor area, there must be positioned immediately next to that area signage complying with subsection (4).

(4)  Signage required by subclause (2) or subclause (3) must---

(a)  state that hazardous substances are present; and

(b)  state that hazardous property of each of them; and

(c)  describe the general type of hazard each of them has; and

(d)  describe,---

(i)     if the substances include explosive substances, the

precautions necessary to prevent unintended explosion of an explosive

substance; and

(ii)    if the substances include flammable substances, the

precautions necessary to prevent unintended ignition of a flammable substance;

and

(iii)   if the substances include oxidising substances or organic

peroxides, the precautions necessary to prevent unintended combustion,

acceleration of a fire, or thermal decomposition of an oxidising substance or organic peroxide.

Regulation 52(1)(c): revoked, on 26 March 2004, by regulation 14 of the

Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

Advertising toxic substances

53 Advertising corrosive and toxic substances

(1)  Subclause (2) applies to a substance if it is---

(a)  a class 8.2 skin corrosive substance; or

(b)  a class 8.3 eye corrosive substance; or

(c)  a class 6.1 acutely toxic substance.

(2)  Every advertisement for a substance to which this subclause applies must contain the information required by regulations 8 and 14.

Regulation 53(1): substituted, on 26 March 2004, by regulation 15 of the

Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

Schedule 1                 r 35(3)(c)

Distances for previewing and hearing information,

documentation, and signage

Quantity of hazardous substance         Distance for priority    Distance for

identifiers              secondary

identifiers

LE 20 g or 20 mL                        0.3 m                    0.2 m

GT 20 g or 20 mL to LE 5 kg or 5 L      0.3 m                    0.3 m

GT 5 kg or 5 L to LE 450 kg or 450 L    1 m                      0.5 m

GT 450 kg or 450 L                      10 m                     10 m

Schedule 2                 rr 37, 38

Quantities requiring documentation

Hazard classification         Description                 Quantity

The following class 1:        gas, liquid, or solid       5 kg (gross

weight) per

package

(a) airbag initiators and

seatbelt pretensioners of

classes 1.4G or 1.4S; and

(b) cable cutters of class 1.4S

(UN 0070); and

(c) power device cartridges of

class 1.4S (UN 0323); and

(d) signal or shock tubes of

class 1.4S (UN 0349); and

(e) cassette degradation devices

of class 1.4S (UN 0432); and

(f) emergency flares and

signalling devices of

classes 1.3G, 1.4G, and 1.4S;

and

(g) model rocket motors of

class 1.4G and 1.4S; and

(h) igniting fuzes of class

1.4G (UN 0317); and

(i) igniters of class 1.4S (UN

0454).

The rest of class 1 except--- gas, liquid, or solid       any quantity

(a)                           fireworks subject to the

Hazardous Substances

(Fireworks) Regulations

2001; and

(b)                           safety ammunition,

including pre-primed

cartridges and primers, of

class 1.4S.

2.1.1A                        non-permanent gas permanent 0.2 kg

gas                         0.2 m³

2.1.1B                        non-permanent gas permanent 0.5 kg

gas                         0.5 m³

2.1.2A                        aerosol                     1.0 L aggregate

water capacity

3.1A                          liquid                      0.5 L

3.1B                          liquid                      1.0 L

3.1C                          liquid                      5.0 L

3.1D                          liquid                      50 L

3.2A, 3.2B, 3.2C              liquid                      any quantity

4.1.1A                        solid                       0.5 kg

4.1.1B                        solid                       3.0 kg

4.1.2A, 4.1.2B, 4.1.2C,       liquid or solid             any quantity

4.1.2D, 4.1.2E, 4.1.2F,

4.1.2G

4.1.3A, 4.1.3B, 4.1.3C        liquid or solid             any quantity

4.2A                          liquid or solid             any quantity

4.2B                          solid                       0.5 kg

4.2C                          solid                       3.0 kg

4.3A                          liquid or solid             any quantity

4.3B                          liquid                      0.5 L

solid                       0.5 kg

4.3C                          liquid                      1.0 L

solid                       1.0 kg

5.1.1A                        liquid or solid             any quantity

5.1.1B                        liquid                      0.5 L

solid                       0.5 kg

5.1.1C                        liquid                      1.0 L

solid                       1.0 kg

5.1.2A                        non-permanent gas permanent 1.0 kg

gas                         1.0 m³

5.2A                          liquid or solid             any quantity

5.2B, 5.2C, 5.2D, 5.2E,       liquid or solid             any quantity

5.2F---if substance needs

temperature control under the

Hazardous Substances (Classes

1 to 5 Controls) Regulations 2001

5.2B, 5.2C---if substance     liquid                      25 mL

does not need temperature     solid                       0.1 kg

control under the Hazardous

Substances (Classes 1 to 5

Controls) Regulations 2001

5.2D, 5.2E, 5.2F---if         liquid                      125 mL

substance does not need       solid                       0.5 kg

temperature control under the

Hazardous Substances (Classes

1 to 5 Controls) Regulations 2001

5.2G                          liquid                      50 L

solid                       50 kg

6.1A, 6.1B                    gas, liquid, or solid       any quantity

6.1C, 6.6A, 6.7A, 6.8A, 6.9A  non-permanent gas           0.1 kg

permanent gas               0.1 m³

liquid                      0.1 L

solid                       0.5 kg

6.1D, 6.5A, 6.5B, 6.6B, 6.7B, non-permanent gas           1.0 kg

6.8B, 6.8C, 6.9B              permanent gas               1.0 m³

liquid                      1.0 L

solid                       3.0 kg

6.1E, 6.3A, 6.3B, 6.4A        liquid                      50 L

solid                       50 kg

8.2A                          gas, liquid, or solid       any quantity

8.2B                          non-permanent gas           0.5 kg

permanent gas               0.5 m³

liquid                      0.5 L

solid                       1.0 kg

8.1A, 8.2C, 8.3A              non-permanent gas           1.0 kg

permanent gas               1.0 m³

liquid                      1.0 L

solid                       2.0 kg

9.1A, 9.1B, 9.1C, 9.2A, 9.2B, liquid                      5.0 L

9.2C, 9.3A, 9.3B, 9.3C, 9.4A, solid                       5.0 kg

9.4B, 9.4C

9.1D, 9.2D                    liquid                      50 L

solid                       50 kg

(All liquid volumes measured at 20oC)

Schedule 2: substituted, on 26 March 2004, by regulation 16

of the Hazardous Substances (Identification) Amendment

Regulations 2004 (SR 2004/45).

Schedule 3                         r 51

Quantities requiring signage

Hazard classification    Description                     Quantity

1                        Fireworks subject to the        1 000 kg (gross

Hazardous Substances (Fireworks)weight)

Regulations 2001

Safety ammunition, including    10 000 kg (gross

pre-primed cartridges and       weight)

primers, of class 1.4S

Airbag initiators and seatbelt  5 000 kg (gross

pretensioners of class 1.4G and weight)

1.4S

Cable cutters of class 1.4S (UN 5 000 kg (gross

0070)                           weight)

Power device cartridges of class5 000 kg (gross

1.4S (UN 0323)                  weight)

Signal or shock tubes of class  5 000 kg (gross

1.4S (UN 0349)                  weight)

Cassette degradation devices of 5 000 kg (gross

class 1.4S (UN 0432)            weight)

Propellants of class 1.1C (UN   50 kg

0160) and 1.3C (UN 0161 and

UN 0499), gun-powder of

class 1.1D (UN 0027), and

substances of classes 1.3G,

1.4G, and 1.4S not listed above

All remaining explosive         any quantity

hazardous classifications

2.1.1A                   non-permanent gas               250 kg

permanent gas                   100 m³

2.1.1B                   non-permanent gas               500 kg

permanent gas                   200 m³

2.1.2A                   aerosol                         3 000 L aggregate

water capacity

3.1A, 3.2A, 4.1.3A,      liquid                          50 L

4.2A, 4.3A               solid                           50 kg

3.1B, 3.2B, 4.1.3B,      liquid                          250 L

4.2B, 4.3B               solid                           250 kg

3.1C, 3.2C, 4.1.3C,      liquid                          1 000 L

4.2C, 4.3C               solid                           1 000 kg

3.1D                     liquid                          10 000 L

4.1.1A                   solid                           250 kg

4.1.1B                   solid                           1 000 kg

4.1.2A, 4.1.2B           liquid                          50 L

solid                           50 kg

4.1.2C, 4.1.2D           liquid                          250 L

solid                           250 kg

4.1.2E, 4.1.2F, 4.1.2G   liquid                          1 000 L

solid                           1 000 kg

5.1.1A                   liquid                          50 L

solid                           50 kg

5.1.1B                   liquid                          500 L

solid                           500 kg

5.1.1C                   liquid                          1 000 L

solid                           1 000 kg

5.1.2A                   non-permanent gas               250 kg

permanent gas                   500 m³

5.2A, 5.2B               liquid                          1 L

solid                           1 kg

5.2C, 5.2D, 5.2E, 5.2F   liquid                          10 L

solid                           10 kg

6.1A                     liquid                          50 L

solid                           50 kg

6.1B                     liquid                          250 L

solid                           250 kg

6.1C                     liquid                          1 000 L

solid                           1 000 kg

6.1D                     liquid                          10 000 L

solid                           10 000 kg

6.1A, 6.1B, 6.1C         non-permanent gas               5 kg

permanent gas                   2.5 m³

8.1A                     liquid                          1 000 L

solid                           1 000 kg

8.2A                     non-permanent gas               5 kg

permanent gas                   2.5 m³

liquid                          50 L

solid                           50 kg

8.2B                     non-permanent gas               50 kg

permanent gas                   25 m³

liquid                          250 L

solid                           250 kg

8.2C, 8.3A               liquid                          1 000 L

solid                           1 000 kg

9.1A, 9.2A, 9.3A, 9.4A   liquid                          100 L

solid                           100 kg

9.1B, 9.1C, 9.2B, 9.2C,  liquid                          1 000 L

9.3B, 9.4B, 9.4C         solid                           1 000 kg

9.1D, 9.2D, 9.3C         liquid                          10 000 L

solid                           10 000 kg

Schedule 3: substituted, on 26 March 2004, by regulation 16 of the Hazardous

Substances (Identification) Amendment Regulations 2004 (SR 2004/45).

Martin Bell,

Acting for Clerk of the Executive Council.

Issued under the authority of the Acts and Regulations Publication Act 1989.

Date of notification in Gazette: 31 May 2001.

Index to notes

1           General

2           Status of reprints

3           How reprints are prepared

4           Changes made under section 17C of the Acts and Regulations

Publication Act 1989

5           List of amendments incorporated in this reprint (most recent                  first)

Notes

1           General

This is a reprint of the Hazardous Substances (Identification) Regulations

2001.

The reprint incorporates all the amendments to the Hazardous Substances

(Identification) Regulations 2001 as at 9 June 2005, as specified in the list of amendments at the end of this reprint.

2           Status of reprints

Under section 29A of the Evidence Act 1908, reprints are presumed to correctly state, as at the date of the reprint, the law enacted by the principal enactment and by the amendments to that enactment. This presumption applies even though editorial changes authorised by section 17C of the Acts and Regulations Publication Act 1989 have been made in the reprint. For an outline of editorial changes made in reprints, see below.

This presumption may be rebutted by producing the official volumes of statutes or statutory regulations in which the principal enactment and its amendments are contained.

3           How reprints are prepared

The following conventions are followed in the preparation of reprints

generally, and they have been followed, where relevant, in the preparation of this reprint:

o           the enacting words have been omitted

o           provisions that have been repealed or revoked are omitted, but a

note indicates the repealing enactment or revoking provision

o           footnotes are included to indicate where---

o  words, phrases, or provisions have been inserted or substituted,the commencement date of the amend- ment, and the particular

enactment responsible for the change

o  references in a reprinted enactment to any repealed enactment have been replaced with a reference to any enactment that

replaces, or corresponds to, the repealed enactment, in reliance on section 22(2) of the Interpretation Act 1999

o           any term used in a reprinted enactment that has been deemed, by

another enactment, to be read as if it were another term has been replaced by that other term

o           references in a reprinted enactment to amounts in pounds,

shillings, and pence have been replaced by a reference to the

equivalent amount in decimal currency (dollars and cents), in

reliance on section 7 of the Decimal Currency Act 1964

o           where the principal enactment has made a textual amendment to

another enactment, and the amendment has been incorporated in a

reprint of that other enactment, the text of the amendment is not  reprinted in full in the reprint

o  where a provision of an amending enactment has made a textual amendment to the principal enactment, the provision of the

amending enactment is not reprinted.

4           Changes made under section 17C of the Acts and Regulations

Publication Act 1989

Section 17C of the Acts and Regulations Publication Act 1989 authorises the

making of editorial changes in a reprint so that, to the extent permitted, the format and style of the reprinted enactment is consistent with current

legislative drafting practice. Changes that would change the effect of the

legislation are not permitted.

A new format of legislation was introduced on 1 January 2000.

Changes to legislative drafting style have also been made since 1997, and are ongoing. To the extent permitted by section 17C of the Acts and Regulations Publication Act 1989, all legislation reprinted after 1 January 2000 is in the new format for legislation and reflects current legislative drafting practice.

In outline, the editorial changes made in reprints under the authority of

section 17C of the Acts and Regulations Publication Act 1989 are as follows, and they have been followed, where relevant, in the preparation of this reprint:

o           unnecessary referential words (such as "of this section" and

"of this Act") have been omitted

o           typeface and type size have been changed to accord with current

drafting practice (Times Roman, generally in 11.5 point with

13.5 point leading)

o           the setting out of provisions has been changed to accord with

current drafting practice. These changes include---

o  changes to the indenting and layout of provisions

o  the repositioning of section headings, so that the number and

heading appear above the section

o  the reformatting of definitions, so that the defined term

appears in bold type, without quotation marks

o           dates are expressed in a manner consistent with current drafting  practice (eg, a date formerly expressed as "the 1st day of January 1999" is now expressed as "1 January 1999")

o           the date of assent has been repositioned so that it appears on the front page of each Act

o           punctuation is consistent with current drafting practice (eg,

colons are not used after definitions)

o           Parts numbered with roman numerals have been replaced with arabic numerals, and all cross-references changed accordingly

o           the case and appearance of letters and words have been changed to accord with current drafting practice

o           headings have been reformatted (eg, headings where each word

formerly appeared with an initial capital letter followed by small capitals have been changed so that the heading appears in bold,

with only the first word (and any proper names) appearing with an  initial capital)

o           letters appearing in small capitals in section and subsection

references have been changed to capital letters (eg, "section

7---a" is now expressed as "section 7A")

o           schedules have been renumbered (eg, Schedule 1 replaces First

Schedule), and all cross-references changed accordingly

o           running heads (the information that appears at the top of each

page) have been altered to accord with current drafting practice.

Two-column schedules of consequential amendments, and schedules of repeals,

have been reformatted, and rearranged into alphabetical order (rather than

chronological), to accord with current drafting practice.

5           List of amendments incorporated in this reprint

(most recent first)

Hazardous Substances (Identification) Amendment Regulations 2004 (SR 2004/45)

Hazardous Substances (Identification) Amendment Regulations 2003 (SR 2003/180)

The Hazardous Substances (Identification) Regulations 2001 are administered in the Ministry for the Environment.