Sewerage and Drainage Act

(Chapter 294, Sections 51 and 74)

Sewerage and Drainage (Sanitary Works) Regulations

Rg 2

G.N. No. S 167/1999

REVISED EDITION 2007

(15th May 2007)

[1st April 1999]

PART I

PRELIMINARY

Citation

1.  These Regulations may be cited as the Sewerage and Drainage (Sanitary Works) Regulations.

Definitions

2.  In these Regulations, unless the context otherwise requires —

“Code of Practice” means the Code of Practice on Sewerage and Sanitary Works issued under section 32 of the Act;

“common property” has the same meaning as in the Land Titles (Strata) Act (Cap. 158);

“discharge pipe” means a pipe which conveys the discharges from a sanitary appliance or a floor trap;

“discharge stack” means a main vertical discharge pipe;

“sanitary drainage system” means a network of underground pipes comprising drain-lines, branch drain-lines, fittings and inspection chambers for the conveyance of sewage within any premises to a sewerage system;

“sanitary plumbing system” means a system of sanitary pipework above the ground comprising one or more discharge pipes, discharge stacks, ventilating pipes, ventilating stacks and fittings for the conveyance of sewage from any premises to a sanitary drainage system;

“sanitary works” include works on any sanitary plumbing system or sanitary drainage system and the fixing, alteration and repair of a sanitary appliance, and other works connected therewith;

“ventilating pipe” means a pipe provided to limit the pressure fluctuations within any sanitary plumbing system;

“ventilating stack” means a main vertical ventilating pipe.

PART II

SANITARY WORKS

Requirements for sanitary works

3.  All sanitary works shall be carried out in accordance with the Code of Practice and the following requirements:

(a)

all rain water shall, where practicable, be separated from sewage, and all rain water from roofs and open spaces and any overflow of water from storage tanks shall be diverted to a surface storm water drain and away from any opening connected to a sewerage system;

(b)

any drainage system serving common corridors, void decks or multi-storey car parks (except designated wash areas) shall be connected to storm water drains;

(c)

all rain water from open areas in backyards, courtyards, airwells and other paved areas within premises shall be discharged to storm water drains;

(d)

where there is an enclosed court or backyard in the premises with no available surface outlet for rain water, a sufficient area of the surface of the enclosed court or backyard shall be paved so as to be able to lead any rain water therefrom into the drain-line via a gully trap;

(e)

no rain water pipe leading down from the roof of a building shall be utilised as a ventilating stack or discharge stack;

(f)

all sullage water from a service station and motor workshop shall be discharged into a sewerage system via such oil and petrol interceptor as may be approved by the Board;

(g)

all areas used for washing purposes shall be roofed over and all sullage water from such roofed wash areas, refuse chutes or bin centres, and any other place in a building as required by the Board, shall be discharged into a sewerage system;

(h)

all sullage water from food shops, restaurants and eating establishments shall be discharged into the sewerage system via such grease trap as may be approved by the Board;

(i)

all car washing bays, lubrication bays and other similar areas in petrol stations, motor garages or motor workshops shall be roofed over and all sullage water from such places shall be discharged into a sewerage system via such oil and petrol interceptor as may be approved by the Board;

(j)

all pump islands at petrol stations shall be connected to surface storm water drains;

(k)

all backwash water from a swimming pool filter shall be discharged into a sewerage system;

(l)

all open shower points for swimming pools shall be connected to storm water drains; and

(m)

no sanitary plumbing system shall be laid —

(i)

over, above or across any potable water storage tank, electrical transformer or switch gear; or

(ii)

in any place where it can endanger the health or safety of any person.

Permission to work in public sewerage system

4.

—(1)  Notwithstanding that a clearance certificate or the approval of the Board has been obtained under section 33 of the Act in respect of any sanitary works, no person shall enter the public sewerage system to carry out such works except with the prior permission of the Board.

(2)  The Board may, in granting any permission under paragraph (1), impose such conditions as it thinks fit.

PART III

SANITARY APPLIANCES

Sanitary appliance to be provided with flushing cistern, flush valve, etc.

5.

—(1)  Every water closet pan, urinal, bidet or similar sanitary appliance fixed in any premises shall be provided with a flushing cistern, flush valve or other flushing device which shall be continuously supplied with water.

(2)  Every sanitary appliance together with the appurtenances connected thereto which is installed below ground level shall have the sewage therefrom delivered to a sanitary drainage system by means of such sewage ejector, pump or other appliance as may be approved by the Board.

Approval for sanitary appliances, etc.

6.

—(1)  No sanitary appliances, pipes or fittings shall be used in sanitary works unless —

(a)

the appliances, pipes or fittings are certified under any product certification scheme approved by the Board for the purposes of this regulation; or

(b)

the Board has given its approval for the appliances, pipes or fittings to be used in the sanitary works.

(2)  Any person applying for the approval of the Board under paragraph (1)(b) shall —

(a)

submit an application in such form and manner as the Board may require;

(b)

deliver such number of samples of the appliances, pipes or fittings for which approval is sought as the Board may require to any testing laboratory designated by the Board for testing; and

(c)

pay the fees for any test conducted by the testing laboratory.

(3)  Notwithstanding that any appliances, pipes or fittings have been certified in accordance with paragraph (1)(a), such appliances, pipes or fittings shall not be used in any sanitary works if their certification under the certification scheme has been cancelled.

(4)  The Board may withdraw its approval given under paragraph (1)(b) if —

(a)

the sanitary appliances, pipes or fittings are found to be unsafe or unsuitable for use; or

(b)

the design, material or construction of the appliances, pipes or fittings is changed without the approval of the Board.

(5)  Notwithstanding that any appliances, pipes or fittings have been approved under paragraph (1)(b), such appliances, pipes or fittings shall not be used in any sanitary works if the Board has withdrawn its approval under paragraph (4).

PART IV

REGISTERED PLUMBERS

Maintenance and inspection of sanitary plumbing and drainage system

7.

—(1)  The owner of any common property shall —

(a)

ensure that all sanitary plumbing systems and sanitary drainage systems installed or located on the common property are at all times properly maintained, kept in good working order and free from any leakage;

(b)

cause the sanitary plumbing systems and sanitary drainage systems to be thoroughly examined and inspected and certified to be in good working order and free from any leakage by a professional engineer at such intervals as the Board may require; and

(c)

keep a proper record of all certificates issued by the professional engineer under sub-paragraph (b), and produce such records to the Board when required.

(2)  For the purposes of this regulation, “professional engineer” means a professional engineer registered under the Professional Engineers Act (Cap. 253) in the civil or structural engineering discipline or the mechanical engineering discipline.

PART V

MISCELLANEOUS

Penalty

8.  Any person who contravenes any provision of these Regulations shall be guilty of an offence and shall be liable on conviction to a fine not exceeding $5,000 and, in the case of a continuing offence, to a further fine of $500 for every day or part thereof during which the offence continues after conviction.

Rate of interest under section 51 (1) (b) of Act

9.  The rate of interest under section 51(1)(b) of the Act shall be 3.6% per annum.

[G.N. Nos. S 167/99; S 157/2001; S 373/2002; S 581/2006]

LEGISLATIVE HISTORY

Sewerage and Drainage (Sanitary Works) Regulations

(CHAPTER 294, Rg 2)

| This Legislative History is provided for the convenience of users of the Sewerage and Drainage (Sanitary Works) Regulations. It is not part of these Regulations. |  |  |  |
| --- | --- | --- | --- |
| 1. | G. N. No. S 167/1999—Sewerage and Drainage (Sanitary Works) Regulations 1999 |  |  |
|  | Date of commencement | : | 1 April 1999 |
| 2. | 2001 Revised Edition—Sewerage and Drainage (Sanitary Works) Regulations |  |  |
|  | Date of operation | : | 31 January 2001 |
| 3. | G. N. No. S 157/2001—Sewerage and Drainage (Sanitary Works) (Amendment) Regulations 2001 |  |  |
|  | Date of commencement | : | 1 April 2001 |
| 4. | G. N. No. S 373/2002—Sewerage and Drainage (Sanitary Works) (Amendment) Regulations 2002 |  |  |
|  | Date of commencement | : | 1 August 2002 |
| 5. | G. N. No. S 581/2006—Sewerage and Drainage (Sanitary Works) (Amendment) Regulations 2006 |  |  |
|  | Date of commencement | : | 13 October 2006 |
| 6. | 2007 Revised Edition—Sewerage and Drainage (Sanitary Works) Regulations |  |  |
|  | Date of operation | : | 15 May 2007 |